Objectives

- Understand several building blocks used in transistor circuit design and learn about their limitations.

- Reinforce concepts learned in the design of common-source, common-gate, and common-drain amplifiers.

Equipment

- 2 CD4007UBE transistor array ICs

- Breadboard and various wires

- Various resistors

- Digilent Analog Discovery (DAD) Board

Introduction

In this lab we will be using the CD4007UBE transistor array. Each 4007 array has three NMOS and three PMOS transistors. They are arranged in NMOS/PMOS pairs as shown below. Each NMOS/PMOS pair is internally connected slightly differently, which constrains how the transistors can be used in external circuits. Use the following layout in figure 1 below to build your circuits in this lab.

Figure 1 – CD4007UBE Layout

Only a few transistors on each chip will be used. The other FETs we will leave “floating” (disconnected). However, on each chip pins 14 and 7 must always be connected to the positive and negative supply voltages VDD and VSS, respectively. These pins are internally connected to the sources of transistors N1 and P1 but also to the substrates of the other MOSFETs in the chip. The biasing is required so that all of the transistors are isolated by reverse-biased PN junctions.

In this lab we will first be introduced to building current mirrors in Part 1. Then we will build an PMOS circuit in part 1 using one chip and we will then build an NMOS circuit in Part 2. *We will eventually connect the circuits together so be sure to keep them on your breadboard before coming to lab.

Pre-Lab

Part 1: Intro to Current Mirrors (10 Points)

A current mirror is used to copy or multiply the input current. It is often used as a bias circuit that provides a known current to an analog circuit like a common-source amplifier with an active load. A current mirror in CMOS relies on the property that ideal transistors of the same size (W/L) with the same VGS have the same drain current (ID) regardless of VDS as long as both are in saturation. A simple NMOS current mirror is shown in Figure 2. With real transistors, however, ID changes slightly with VDS because of the channel-length modulation. In small signal, this effect appears as a finite output impedance (as opposed to the infinite output impedance of an ideal current mirror).

Figure 2 – NMOS current mirror.

- +Create the following current mirror in Figure 2 on your breadboard and complete the following items. (5 Points)

- Choose RB such that ID1 = 1 mA; (you can use a normal resistor).

- Set VDD = 5 V.

- Use your CD4007UBE array IC and get familiar with its pin-layout.

- Set Vin to a 2 V, 1 kHz sine wave and plot ID2 versus Vo using XY mode in waveforms.

- Show the output swing and output impedance of the current mirror in waveforms as well. (5 Points)

Used a math function to divide voltage by resistance for current

Part 2: Current Mirror Building Block (30 Points)

Current mirrors are fundamental biasing elements used in many analog integrated circuits. By using matched transistors operating in saturation, a current mirror can replicate a reference current in another branch of a circuit. In this section, a PMOS current mirror is constructed using the CD4007 transistor array to investigate how accurately current can be mirrored and how non-ideal transistor effects influence the output current and output resistance.

Figure 3 – PMOS Current Mirror

- Build a PMOS current mirror using the following specifications:

- Use your first 4007 array chip.

- Use the “P2” and “P3” MOSFETs.

- Set V3 to 0 V (DC).

- Choose a value for R2 to set I3 to roughly 0.05 mA (50 uA).

* To calculate I3, use RX = ~1 kΩ as an ammeter to find the current of P3: I3 = VX / RX.

Record your chosen value of R2 (10 Points)

1k for 5v/1k = 5mA

- Measure and record all node voltages and verify that both FETs are active. Are the drain currents of P2 and P3 approximately equal?  (10 Points)

yes

- Add a 400 mV 1 kHz sine wave to V3. Measure the resulting AC component of I3. Estimate the output resistance of the current mirror as the ratio of the change in VD3 to the change in I3. (10 Points)

Part 3: NMOS Differential Pair Building Block (40 Points)

Differential amplifiers are one of the most important building blocks in analog circuit design and form the input stage of many operational amplifiers. A differential pair amplifies the difference between two input signals while rejecting signals that are common to both inputs. In this section, an NMOS differential pair will be implemented using the CD4007 array. The circuit will be used to examine offset voltage, differential gain, common-mode gain, and the common-mode rejection ratio (CMRR).

Figure 4 – NMOS Differential Pair Amplifier

- Build the differential pair amplifier in LTspice with the following:

- Use your second CD4007UBE array IC

- Use the “N2” and “N3” FETs from your NMOS array.

- RSS = 33 kΩ

- RD1 = RD2 = 68 kΩ.

- VSS = – 5 V.

- VDD = 5 V

- Use Waveform Generators 1 and 2 for input voltages V1 and V2.  (4 Points)

- Find a combination of DC values for V1 and V2 so that their sum is approximately 0 V (will be in the mV range), and the output voltages Vo1 and Vo2 are the same and in the range of 3 V. Record VO1, VO2, and the node voltage at the source of the MOSFETs. Calculate the current using the following values: VDD, VO1/2, and RD1/2. (6 Points)

- Because the FETs N2 and N3 are matched, the difference between V1 and V2 when the output voltages are equal should be small (mV range). This difference is the offset voltage VOS. Record your VOS. 	(4 Points)

- With the DC values of V1 and V2 fixed, add a 50 mV 1 kHz sine wave to V1. Measure the amplitudes of Vo1 and Vo2, and observe their phases/signs. Record the amplitude of the source terminals of N2 and N3. (8 Points)

- Turn off the AC part of V1 and add a 50 mV 1 kHz sine wave to V2. Record the same values from IV.  (4 Points)

- Compute the differential mode gain ADM, differential output voltage (Vodm), and differential input voltage (Vidm). ADM can be defined as the ratio of the differential output signal Vodm = Vo1 - Vo2 to the differential input signal Vidm = Vi1 – Vi2, ADM = Vodm/Vidm. (6 Points)

- Add 10 mV 1 kHz sine waves with equal phase to both V1 and V2 with their DC values still unchanged.

Repeat IV. Measure the differential output voltage. Compute the common-mode gain ACM. Where ACM is the ratio of the differential output signal Vodm = Vo1 - Vo2 to the common-mode input signal		      Vicm = Vi1 = Vi2	 ADM  = Vodm / Vicm. (6 Points)

- Compute the common-mode rejection ratio such that CMRR =  20 log10(|ADM|/|ACM|). (4 Points)