Full Name: Evan Baesler

EEL 3135	– Classwork #08

Milestone:

• Complete any 2 questions to receive credit.

Question #1:	Compute the following by hand:

y[n] = (2δ[n + 1] − δ[n − 2] + 3δ[n − 4]) ∗ (δ[n − 1] + 2δ[n − 3])

- Write out all shifted and weighted impulse terms explicitly.

2δ[n+1] * δ[n-1] = 2[n]

2δ[n+1] * 2δ[n-3] = 4δ[n-2]

-δ[n-2] * δ[n-1] = -δ[n-3]

-δ[n-2] * 2δ[n-3] = -2δ[n-5]

3δ[n-4] * δ[n-1] = 3δ[n-5]

3δ[n-4] * 2δ[n-3] = 6δ[n-7]

- Simplify to a single expression for y[n] using discrete convolution properties.

= 2δ[n] + 4δ[n-2] -δ[n-3] + δ[n-5] + 6δ[n-7]

- Plot the resulting y[n] from part (2) in MATLAB for n = −5:10 using the stem() command. Label all nonzero impulses clearly and include proper axis titles.

Question #2:	Two FIR systems are connected in cascade:

with input x[n] = u[n].

- Compute the overall impulse response hT[n] = h1[n] ∗ h2[n].

½ δ[n] – ½δ[n-2] + δ[n-1] – δ[n-3]

- Determine the system output y[n] = x[n] ∗ hT[n].

½u[n] – ½u[n-2] + u[n-1] – u[n-3]

- Is the overall system causal and FIR? Justify your answer.

It is causal because there are no advanced checks (n+1, n+2, n+x)

It is FIR, because it is only non-zero at 0,1,2,3 and then zero.

Question #3:	Given: Let x = [1,−1,2,0] and h = [2,1].

- Write MATLAB code that constructs the Toeplitz matrix for x and computes y = Tx∗h(:).

- Compare the result to conv(x, h) and confirm they match.

- Briefly explain why both methods yield identical outputs according to the commutative property of convolution.

Question #4:	Given: A discrete-time signal is given as:

x[n] = sin(0.2πn) + 0.5w[n],

where w[n] is white Gaussian noise. A length-5 moving average FIR filter is applied:

- Write MATLAB code to generate x[n], apply the filter using conv(x,h,’same’), and plot both signals.

- Estimate qualitatively how the filter affects the frequency content (i.e., explain why it smooths x[n]).

- If the filter length is increased to 20, describe the trade-off between noise reduction and signal delay.