Full Name: Evan Baesler

EEL 3135	– Classwork #05

Milestone:

• Complete any 2 questions to receive credit.

Question #1:	Given:

x(t) = 4ej(10t+π/6) + 4e−j(10t+π/6)

- Use Euler’s identity to convert x(t) into its real sinusoidal form.

Since inverse amplitude is split in half, we multiply it by two.

xt=8cos⁡(10t+π6)

- Identify the amplitude, phase shift, and frequency of the resulting cosine wave.

Amplitude: 8

Phase: π6

Frequency: 102π

- Sketch one period of the waveform in Matlab.

Question #2:	You are given the following frequency-domain information for y(t):

ω0 = 5 rad/s,	a0 = 3,	a±1 = 2e∓jπ/4,	a±2 = 1e∓jπ/3

- Write y(t) as a sum of complex exponentials.

- Rewrite y(t) using cosine terms (real Fourier series).

- Plot y(t) over one period in MATLAB.

Question #3: The figure below shows the two-sided magnitude spectrum of x(t) with frequency components at f = ±25, ±50, ±75 Hz. Each spectral line is labeled with its corresponding complex coefficient ak in rectangular form (real + imaginary).

- Determine the fundamental frequency f0 and fundamental period T0 of x(t).

f0= GCD25,50,75=25 Hz

- Write x(t) in complex exponential form using the coefficients ak from the spectrum.

For +- 75Hz:

1.00-1.73iej75*2πt+1.00-1.73ie-j75*2πt

=1.9982ej75*2πt+1.9982e-j75*2πt

For +- 50Hz:

3.54+3.54iej50*2πt+|3.54+3.54i|e-j50*2πt

=5.0063ej50*2πt+5.0063e-j50*2πt

For +- 25Hz:

2.60-1.50ie-j25*2πt+|2.60-1.50i|e-j25*2πt

=3.0017ej25*2πt+3.0017e-j25*2πt

a-3= 1.9982e-j75*2πt

a-2= 5.0063e-j50*2πt

a-1= 3.0017e-j25*2πt

a0= 0

a1= 3.0017ej25*2πt

a2= 5.0063ej50*2πt

a3= 1.9982ej75*2πt

- Use Euler’s formula to rewrite x(t) as a sum of cosines in the time domain.

Total:

6.0033cos(50πt-0.5233)+10.0126cos⁡(100πt+0.7854)+3.9964cos⁡(150πt-1.0467)

Question #4:	Given

,

- Evaluate.

- Express y(t) in complex exponential form using Euler’s formula.

- Sketch the magnitude spectrum of y(t) at the harmonic frequencies kω0 using Matlab.