# OBJECTIVES

• Understand the design and operation of a common-source (CS) amplifier.

# EQUIPMENT

- 2 each: 2N7000 NMOS FET

- Multiple capacitors

- Various resistors

- Breadboard and various wires

- Digilent Analog Discovery (DAD) Board

INTRODUCTION

Figure 1

–

MOSFET amplifi

er

INTRODUCTION

Figure 1

–

MOSFET amplifi

er

- Drain Resistance

In this lab we will manipulate the drain resistor (RD=220 Ω) in circuits like the one above to perform certain functions. For this lab, this includes measuring drain current (ID) and serving as the load resistor in a common-source amplifier.

- Testing/Tools

We will test the circuits two ways:

- With the waveform generator as a DC voltage source

- With a sine wave superimposed on a DC bias voltage.

We will use the scope in two ways as well:

- As a DC voltmeter

- In XY mode to plot Vout vs. Vin

- Structure

We will first characterize the current-voltage behavior of your MOSFET and then choose a good operating point for the circuit as an amplifier. Then we will characterize the circuit as an amplifier. Finally, we will add a circuit using a negative power supply to set a more stable bias current and characterize the improved circuit.  All the circuits will be built using variations on same setup in figure

# In-Lab

## Part 1: DC-Degenerated Common-Source Amplifier (20 Points)

The amplifier requires a carefully selected DC bias voltage to be added to the input signal. A different FET would likely require a different value. To make the current less dependent on MOSFET parameter variations, we can use a circuit like the one shown below using the negative power supply and a “degeneration” resistor RS1 = VSS / IDQ (previous current)

- Build the following circuit in Figure 2 on your breadboard using a 2N7000 Transistor and a RS1 = 680 Ω., RD = 220 Ω. (4 Points)

Figure 2 – Common-source amplifier with degeneration.

- Measure the effect of RS1 on the voltage transfer curve. Set up WaveGen1 to supply a 10 kHz sinusoid with peak to peak voltage from 1 V to 4 V. Set up the scope in XY mode with Channel 1 (VG) on the x-axis and Channel 2 (VD) on the y-axis. Adjust the input offset and amplitude and the scope sensitivities to get a nice VTC display and insert a screenshot. (2 Points)

- Using the same setup, swap your 2N7000 for a different one and plot the resulting VTC. Compare the sensitivity to swapping the FET. How does including RS affect the sensitivity to the FET parameters compared to when it wasn’t included in the prelab? (2 Points)

It makes the switch of the FETS not different as it did in the pre-lab.

Figure 3 – Common-source amplifier with degeneration (with Bypass capacitor).

- Including RS reduces (degenerates) the voltage gain. To reobtain the voltage gain, add a large-valued bypass capacitor CS1 in parallel with RS1. We’ll make CS1’s value and the input frequency large enough that CS1 won’t have time to charge or discharge much during half cycles of the input.        Add CS1 = ~22 uF. Electrolytic capacitors are polarized; connect the + terminal of your capacitor to the higher voltage, not to VSS. Obtain your VTC. (4 Points)

- With WaveGen1, apply a 50 mV 10 kHz sine wave with DC offset = VGQ  and for VG. * If the output signal doesn’t look sinusoidal, reduce the input signal amplitude. Measure the amplitudes of the input and output sinusoids vo and vi. Save these waveforms for your report. Compute the voltage gain Av = vo / vi.  (4 Points)

Vi = 55.147mVpp, Vo = 342.69mVpp, Av = 342.69/55.147 = 6.21

- Use the network analyzer to obtain the frequency response of the circuit. Identify the frequency (fz) at which the gain enhancement due to the capacitor begins and the frequency fL where the gain reaches its maximum (mid-band) value. Also note the upper cutoff (-3dB) frequency fH. (3 Points)

Fz = Around 25Hz

FL = Around 500Hz

FH = Around 7-8MHz

- What would happen to frequencies fZ and fL if you used a higher-value capacitor for CS1. (1 Point)

They will both go to lower frequencies as we increase CS1

## Part 2: AC-Coupled Common-Source Amplifier (3 Points) (Optional)

In this part, the amplifier is modified to use AC coupling at both the input and output while establishing the MOSFET’s DC gate bias with a resistor voltage divider. This configuration allows the amplifier to process small AC signals without disturbing the chosen operating point. This improves signal integrity and makes the circuit more practical for real-world applications.

- Build the following circuit in Figure 3 on your breadboard using a 2N7000 Transistor, RS1 = 680 Ω,

RD = 220 Ω, CD1 = 1 uF, CG1 = 1 uF, and CS1 = 22 uF. Choose values for RB1 and RB2 to set VG = VGQ, while keeping their parallel combination > 100 kΩ. (1 Point)

Figure 4 – AC-coupled common-source amplifier.

- Put WaveGen1 in DC mode and slightly shift VG while observing VG and VD. If it’s working right, the circuit should return to your operating point after every change. Apply a 50 mV 10 kHz sine wave for

VG. As before, if the output signal doesn’t look sinusoidal, reduce the input signal amplitude. (1 Point)

- Measure the amplitudes of the input and output sinusoids. Take note of these waveforms for your report. Compute the mid-band voltage gain. (1 Point)