REQUIREMENTS NOT MET

N/A

PROBLEMS ENCOUNTERED

N/A

HOMEWORK EXERCISES

- ATMEL STUDIO INSTALLATION

Successfully installed Microchip Studio 7.0! (See Appendix)

- LEARNING HOW TO USE ATMEL STUDIO

Did exercises specified (See Appendix for required photos)

PSEUDOCODE/FLOWCHARTS

SECTION 1: Provided Code

PSEUDOCODE:

Include ATxmega1228A1Udef.inc file

Initialize program constants/variables

Org 0

Short jump to MAIN FUNCTION

ORG to $100

MAIN FUNCTION:

; LED is PORTD_DIRSET

Set LED to BIT456

Set LED to RED

Set LED to GREEN

Set LED to BLUE

Set LED to WHITE

Set LED to PINK/MAGENTA

Set LED to BLACK

Clear BIT6 from LED (turning ~BLUE on from active-low behavior)

Set BIT6 from LED (turning BLACK from active-low behavior)

LOOP FUNCTION:

Toggle LED (with stored BIT6, turning on and off forever.)

Short jump to LOOP FUNCTION

PROGRAM CODE

SECTION 1: Provided Code

/****

* GPIO_Output.asm

*

*  Modified: 29 Jan 2026

*  Author: Dr. Schwartz

*  Student: Evan Joseph Baesler

This program shows how to initialize a GPIO port on the Atmel

(Ports D for this example) and demonstrates various ways to write to

a GPIO port. The output will blink LEDs at the bottom left of the

uPAD, labeled D4. PortD4, PortD5, and PortD6 are the red, green,

and blue LEDs, respectively. Note that these LEDs are active-low.

PortC, not used in this example, has eight active-low LEDs on the

Switch/LED backpack. It also has eight switch circuits on PortA that each

use pull-up resistors.

It should be noted that PortD2 (an XMEGA input) and PortD3 (an XMEGA output)

are used for the EDBG (embedded debugger) and outputs to these pins should

be avoided. These pins are intialized in the bootloader code that runs before

any other code.

****/

;Definitions for all the registers in the processor. ALWAYS REQUIRED.

;View the contents of this file in the Processor "Solution Explorer"

;   window under "Dependencies"

.include "ATxmega128A1Udef.inc"

.equ	BIT4 	= 0b00010000

.equ	INV4 	= 0b11101111

.equ	RED = INV4

.equ	BIT5 = 0b00100000

.equ	INV5 = ~BIT5

.equ	GREEN = ~(BIT5)

.equ	BIT6 = 0x40

.equ	BLUE = ~(BIT6)

.equ	BIT456 = 0x70

.equ	WHITE = ~(BIT456)

.equ	BIT64 = 0x50

.equ	PINK = ~(BIT64)

.equ	BLACK = 0xFF

; Any necessary initializations for SRAM in data space?

.ORG 0

;Skip address through 0x00FD (interrupt vectors)

rjmp MAIN

;Start program at 0x0100 so we don't overwrite vectors (0-0x00FD)

.ORG 0x0100

MAIN:

; initialize the data direction of the three LEDs as outputs (PD4-6)

ldi r16, BIT456

sts PORTD_DIRSET, r16

; Notice that the 3 LEDs (RED, GREEN, and BLUE) are all now on, creating white

; This is because these LEDs are active-low & the default value for outputs is low

; The following code shows different ways to write to the GPIO pins.

; Turn on each of the primary colored LEDs in turn, then use some combinations

; These instructions sends the value in r16 to the PORTD pins.

; Since the LEDs are wired as active-low,

;    r16 = RED = 0xEF = 0b1110 1111 will turn the RED LED on.

ldi	r16, RED

sts PORTD_OUT, r16

; Turn on the green LED (only)

ldi	r16, GREEN

sts PORTD_OUT, r16

; The below line could replace the above line if not for PortD3	was not an output

;	sts PORTD_OUT, r16

; Turn on the blue LED (only)

ldi	r16, BLUE

sts PORTD_OUT, r16

; Turn on all of the colored LEDs to make white

ldi	r16, WHITE

sts PORTD_OUT, r16

; Turn on blue and red to make a pink (magenta)

ldi	r16, PINK

sts PORTD_OUT, r16

; Turn off all of the LEDs (i.e., make it black)

ldi	r16, BLACK

sts PORTD_OUT, r16

; Turn on the blue LED (only)

; Notice that the OUTCLR makes the (blue) LED go on,

;   since the LED is active-low

ldi	r16, BIT6			; BIT6 = ~BLUE

sts PORTD_OUTCLR, r16

; Turn off the blue LED (i.e., make BIT6 a 1, using OUTSET)

; Notice that the OUTSET makes the (blue) LED go off,

;  since the LED is active-low

ldi	r16, BIT6

sts PORTD_OUTSET, r16

; Notice that the OUTTGL toggles the value of a PORT pin

;   (in this case the BLUE LED)

LOOP:

sts PORTD_OUTTGL, r16

rjmp LOOP				;repeat forever!

APPENDIX

Figure 1: Microchip Studio simulating the required code, name shown below Author line.

Figure 2: Microchip Studio running the required code on our OOTB board, name shown below Author line.