REQUIREMENTS NOT MET

None.

PROBLEMS ENCOUNTERED

None Encountered.

PSEUDOCODE/FLOWCHARTS

SECTION 1

#define TABSIZE 16

#define OUT_TAB_START 0x3700

#include <avr/io.h>

int main(void){

// create an array of 8-bit value with a size of 16; replace ?? with the values in Table 1, separated by commas

uint8_t IN_TAB [TABSIZE] = { data };

uint8_t *IN_TAB_P = &IN_TAB; // create a pointer to this table

// Note that using the // create a pointer to this table & above is not necessary since by default, 8-bit arrays are

// 	pointers which point to the first element of the array.

// Thus, using both *IN_TAB_P = &IN_TAB;   or   *IN_TAB_P = IN_TAB; will produce the same

// 	functionality since IN_TAB  is an address; however, to remain consistent we will use the  ampersand.

// Initialize a pointer to point to the output table at location 0x3700. Call it OUT_TAB_P

uint8_t OUT_TAB_P (pointer type)value OUT_TAB_START

SECTION 2

//To start the processing, we can use the following template:

while(1){

// Loop till null, i.e., loop while the values at that address are non-zero

// (null)

while(*IN_TAB_P){

// Check for the conditions

// Recall when using an *, we are analyzing the value at that

// pointer

// Dereference IN_TAB_P and check if bit 5 is set (See the Example

// 2).

// if bit 5 was set, then multiply by 2

if( value of IN_TAB_P & (1<<5) not 0 ){

value of IN_TAB_P = value of IN_TAB_P * 2

// check if the value at IN_TAB_P times two is greater than

// or equal to 70

if( value of IN_TAB_P >= 70 ) {

/*Store the result to address pointed by OUT_TAB_P */

// remember to alter the data at an address using a

// pointer, then you must use an (*). (See Example 1)

value of OUT_TAB_P = value of in IN_TAB_P

/*Increment OUT_TAB_P to move to next available space

*/

OUT_TAB_P += 1

}

}

else{

// if the input value (dereference IN_TAB_P) +3 is less than

//78

store the value to OUT_TAB_P

if( value of IN_TAB_P < 75 ){ // Same as IN_TAB_P+3 < 78

/*Store the result to address pointed by OUT_TAB_P */

value of OUT_TAB_P = (value of IN_TAB_P + 3)

/*Increment OUT_TAB_P to move to available space */

OUT_TAB_P += 1

}

}

IN_TAB_P+=1;  // increment IN_TAB_P, the next address in IN_TAB

}

}

return 0;

}

PROGRAM CODE

SECTION 1

/*

* HW3.c

*

* Created: 7/11/2026 3:25:39 PM

* Author : Evan Baesler

*/

#define TABSIZE 16

#define OUT_TAB_START 0x3700

#include <avr/io.h>

int main(void){

// create an array of 8-bit value with a size of 16; replace ?? with the values in Table 1, separated by commas

uint8_t IN_TAB [TABSIZE] = { 201, 0x22, 051, 91, 0x91, 0x2A, 'U', 0130, '#', 0b01010101, 'J', 0x21, 0x55, 32, 0b00011110, 0 };

uint8_t *IN_TAB_P = &IN_TAB; // create a pointer to this table

// Note that using the // create a pointer to this table & above is not necessary since by default, 8-bit arrays are

//  pointers which point to the first element of the array.

// Thus, using both *IN_TAB_P = &IN_TAB;   or   *IN_TAB_P = IN_TAB; will produce the same

// 	functionality since IN_TAB  is an address; however, to remain consistent we will use the  ampersand.

// Initialize a pointer to point to the output table at location 0x3700. Call it OUT_TAB_P

uint8_t *OUT_TAB_P = (uint8_t *)OUT_TAB_START; // Treats the pointer as a hex number

SECTION 2

//To start the processing, we can use the following template:

while(1){

// Loop till null, i.e., loop while the values at that address are non-zero (not null)

while(*IN_TAB_P){

// Check for the conditions

// Recall when using an *, we are analyzing the value at that pointer

// Dereference IN_TAB_P and check if bit 5 is set (See the Example 2).

//if bit 5 was set, then multiply the value by two!

if( *IN_TAB_P & (1<<5) ){

*IN_TAB_P = *IN_TAB_P * 2;

// check if the value at IN_TAB_P times two is greater than or equal to 70

if( *IN_TAB_P >= 70 ) {

/*Store the result to address pointed by OUT_TAB_P */

// remember to alter the data at an address using a pointer, then you must use an (*). (See Example 1)

*OUT_TAB_P = *IN_TAB_P;

/*Increment OUT_TAB_P to move to next available space */

OUT_TAB_P += 1;

}

}

else{

// if the input value (dereference IN_TAB_P) +3 is less than 78, store the result in the output table.

if( (*IN_TAB_P) < 75 ){

/*Store the result to address pointed by OUT_TAB_P */

*OUT_TAB_P = (*IN_TAB_P + 3);

/*Increment OUT_TAB_P to move to available space */

OUT_TAB_P += 1;

}

}

IN_TAB_P+=1;  // increment IN_TAB_P, the next address in IN_TAB

// should this instruction be somewhere else?

// Yes, we moved it up two rows INTO the loop.

}

}

return 0;

}

APPENDIX

Figure 1: Output Table Filtered Values