REQUIREMENTS NOT MET

None

PROBLEMS ENCOUNTERED

None Encountered

FUTURE WORK/APPLICATIONS

This lab taught us how to do communications between microprocessors and other devices! Pretty important to know how to send data to and from an MCU to a peripheral and vice versa, as a microprocessor with no data is not useful at all. Could be used to read data from sensors, send controls to peripheral devices, and more, maybe a lighting controller .

PRE-LAB EXERCISES

- The sampling rate of a UART receiver is usually faster than the baud rate of the overall system. Why is this so?

Because the processor oversamples to detect when the start bit occurs, and to obtain proper sampling of the incoming bits. UART is Asynchronous which means there is not a shared clock; so oversampling helps to sample each bit at the most-accurate time.

- What is the maximum possible baud rate for asynchronous communication within the USART system of the ATxmega128A1U, assuming that the microcontroller has a system clock frequency of 2 MHz and that the USART “double-speed mode” is disabled (i.e., the relevant bit CLK2X is set to 0)? In addition to the maximum rate, provide the values of the relevant registers used to configure that rate. Whenever appropriate, support your answer with calculations.

We would use Table 23-1, which shows for a BSCALE >= 0 we need to use:

fBAUD=fPER2BSCALE*16BSEL+1

BSCALE CAN be -7 to 7 (but a constraint of the equation above is BSCALE >= 0) and BSEL can be any value from 0 to 4095; the smaller we make the denominator the higher the baud frequency; so we make BSCALE = 0, and we make BSEL = 0, leaving:

fBAUD=200000016=125000 bits per second

- In the context of the USART system within the ATxmega128A1U, how many buffers (i.e., memory locations that store temporary data) are used by a transmitter? How many are used by a receiver? Additionally, for both transmitters and receivers, explain how the use of buffers provides greater flexibility to an application involving these components.

The transmitter uses a single write buffer (DATA), which lets it have continuous data transmission without any delay by cycling out the current data for the next; the receiver uses a ‘two-level’ receive buffer (DATA) to ensure that the CPU has time to read the data. This ensures that the CPU has more time to read the data and prevent missing any communications.

- If an asynchronous serial communication protocol of 7 data bits, one start bit, one stop bits, odd parity, and baud rate of 37.44 kHz was chosen, calculate how many seconds it would take to transmit the ASCII character string “Dr. Schwartz saw seven slick slimy snakes slowly sliding southward.” (This string has 67 characters.) Note that ASCII is a 7-bit (not an 8-bit) code. Show all work.

37.44kHz = 37440 bits per second; if it takes 7 bits of data, 1 bit for start, 1 bit for stop, and one bit for parity for one full character read, we simply do:

(10 bits*67 characters)/37740 bits per second = 17.753ms

PSEUDOCODE/FLOWCHARTS

Lab5_2:

MAIN:

Initialize SP

Rcall USART_INIT

LOOP:

Load R17 with ‘U’

Rcall OUT_CHAR

rjmp LOOP

; SUBROUTINES

USART_INIT:

Push r16

; Use PortD

Set the TxD pin high (PORTx_OUT)

Set the TxD pin as output

Select BSCALE and BSEL in BAUDCTRLA/B for 37,440 bps

; for an awkward baud rate like this, we should use negative BSCALE, and go to the smallest increment possible, which is 2^-7, If we do the math we find that:

200000037440=162BSCALE*BSEL+1

53.418816=2BSCALE*BSEL+1

2.338675=2BSCALE*BSEL

2.3386752-7=299=BSEL, which matches the Excel Calculator

Set the frame format / mode in CTRLC

Enable the transmit/receive in CTRLB

Set up Interrupts in CTRLA

Pop r16

ret

OUT_CHAR:

Push r16

; We will first check if there is a transmission in progress, by checking the TXCIF flag in the STATUS register. (bit 6)

WAIT:

Load TXCIF flag in STATUS register, check bit 6  is 1, else branch to WAIT

Load USARTD0_DATA with R17

Pop r16

Ret

Lab5_3:

Same as Lab5_2 but using PORTC instead of PORTD

Lab5_4:

Include prior OUT_CHAR, USART_INIT subroutines from Lab5_2

.org 0

Rjmp MAIN

.org 0x2000

STRING_ADDR:

; input table data here for string

MAIN:

Initialize Stack Pointer

Rcall USART_INIT

Set Z to point to STRING_ADDR

LOOP:

Rjmp LOOP

; --SUBROUTINES-- ;

OUT_STRING:

Push r16

Lpm r17, Z+

Test r17 for zero flag

Branch if not equal to SKIP

If it IS zero we will reset pointers

SKIP: ; No zero flag so we continue writing the string

Rcall OUT_CHAR

Pop r16

Ret

Lab5_5:

; Include the subroutines: OUT_CHAR, USART_INIT (for PORTD) and the associated code.

MAIN:

Set up SP

Call USART_INIT

LOOP:

Call IN_CHAR

Rjmp LOOP

; SUBROUTINES:

; Use subroutines mentioned above from Lab5_4

IN_CHAR:

Push r16

WAIT_IN:

Load status flags

Check if the receive flag is set

Rjmp to WAIT_IN if not

Load r17 with USARTD0_DATA

Rcall OUT_CHAR

Pop r16

Ret

Lab5_6:

; Move over subroutines from Lab5_5, with modified IN_CHAR shown below and new IN_STRING

.equ STR_SIZE = 64

.org 0

Rjmp MAIN

.dseg

.org 0x1000

STR_STORAGE: .byte STR_SIZE

.org 0x100

MAIN:

Initialize SP

Call USART_INIT

Initialize Pointer Y to dseg 0x1000

LOOP:

Call IN_STRING

Rjmp LOOP

; SUBROUTINES

IN_STRING:

Push r16

LOOP_INSTR:

Rcall IN_CHAR

Check for Carriage Return, if true branch to DONE_STRING

Check for Backspace, if true branch to BACK_SPACE

Check for Delete, if true branch to BACK_SPACE

Store r17 to y, post-increment

Rcall OUT_CHAR

Rjmp LOOP_INSTR ; continue loop, ignore conditional statements below

BACK_SPACE:

Sbiw Y, 1

Rjmp LOOP_INSTR

DONE_STRING:

Ldi r17, 0x00

St Y, r17

Pop r16

ret

OUT_STRING:

Push r16

LOOP_OUT:

; Set Y to beginning of string memory

Ld r17, Y+

Tst r17

Branch to DONE_OUT if null

Rcall OUT_CHAR

Rjmp LOOP_OUT

DONE_OUT:

Pop r16

ret

Lab5_7:

; Use USART initialization from previous sections, add interrupt initialization in CTRLA; add a TC

.org 0

Rjmp MAIN

.org USART_INT_VECTOR

Rjmp INTERRUPT

MAIN:

Set PD6 (BLUE_PWM) as an output

Initialize SP

rcall USART_INIT

rcall USART_INTERRUPT_INIT

sei

LOOP:

Toggle PD6 Output

Rjmp LOOP

; SUBROUTINES & INTERRUPTS

PMIC_INIT:

Push r16

Enable the PMIC for low level interrupts

Pop r16

Ret

INTERRUPT:

Push r16

Lds r16, CPU_SREG

Push r16

Rcall IN_CHAR

Rcall OUT_CHAR

Pop r16

Sts CPU_SREG, r16

Pop r16

reti

PROGRAM CODE

Lab5_2:

;

; Lab5.asm

;

; Created: 7/4/2026 3:48:55 PM

; Author : Evan Baesler

;

.include "ATXmega128A1Udef.inc"

.equ B_SEL = 299

.equ B_SCALE = -7

.org 0

rjmp MAIN

.org 0x100

MAIN:

ldi r16, low(RAMEND)

sts CPU_SPL, r16

ldi r16, high(RAMEND)

sts CPU_SPH, r16

rcall USART_INIT

LOOP:

ldi r17, 'U'

rcall OUT_CHAR

rjmp LOOP

; --SUBROUTINES-- ;

USART_INIT:

push r16

; Set the TxD pin high (PORTx_OUT) (PORTC3)

ldi r16, (1<<3)

sts PORTD_OUT, r16

; Set the TxD pin as output

sts PORTD_DIRSET, r16

; Select BSCALE and BSEL in BAUDCTRLA/B for 37,440 bps

; Math Shown on PDF in Pseudocode, derived for BSCALE = -7, BSEL = 299

; BAUDCTRLA = BSEL[7:0]

; BAUDCTRLB = BSCALE[3:0] : BSEL[11:8]

; BSCALE = 1001 = -7 in 2's Comp

; BSEL = 256 + 32 + 8 + 2 + 1 = 2^8 + 2^5 + 2^3 + 2^1 + 2^0

; BAUDCTRLA = 00101011

; BAUDCTRLB = 10010001

ldi r16, low(B_SEL)

sts USARTD0_BAUDCTRLA, r16

ldi r16, ((B_SCALE << 4) | high(B_SEL))

sts USARTD0_BAUDCTRLB, r16

; Set the frame format / mode in CTRLC

; In NON-SPI:

; CTRLC = CMODE[1:0] : PMODE[1:0] : SBMODE : CHSIZE[2:0]

; CMODE = 00 for Asynchronous

; PMODE = 10 for Even Parity

; SBMODE = 0 for 1 Stop Bit

; CHSIZE = 011 for 8-bit data

ldi r16, (USART_CMODE_ASYNCHRONOUS_gc | USART_PMODE_EVEN_gc | USART_CHSIZE_8BIT_gc)

sts USARTD0_CTRLC, r16

; Enable the transmit/receive in CTRLB (CTRLB[4:3]) 4 = RXEN, 3 = TXEN

lds r16, USARTD0_CTRLB

ori r16, 0b00001000 ; Enable Rx/Tx without changing previous defaults

sts USARTD0_CTRLB, r16

; Set up Interrupts in CTRLA (NOT UTILIZED HERE)

pop r16

ret

OUT_CHAR:

push r16

WAIT:

; We check the STATUS register for DREIF, which tells us the current

; transmission is done when the flag goes high. (bit 5)

lds r16, USARTD0_STATUS

sbrs r16, USART_DREIF_bp

rjmp WAIT

sts USARTD0_DATA, r17 ; Data input from main loop

pop r16

ret

Lab5_3:

/*

* Testing.asm

*

*  Created: 7/4/2026 6:47:03 PM

*   Author: Evan Baesler

*/

.include "ATXmega128A1Udef.inc"

.equ B_SEL = 299

.equ B_SCALE = -7

.org 0

rjmp MAIN

.org 0x100

MAIN:

ldi r16, low(RAMEND)

sts CPU_SPL, r16

ldi r16, high(RAMEND)

sts CPU_SPH, r16

rcall USART_INIT

LOOP:

ldi r17, 'U'

rcall OUT_CHAR

rjmp LOOP

; --SUBROUTINES-- ;

USART_INIT:

push r16

; Set the TxC pin high (PORTx_OUT) (PORTC3)

ldi r16, (1<<3)

sts PORTC_OUT, r16

; Set the TxD pin as output

sts PORTC_DIRSET, r16

; Select BSCALE and BSEL in BAUDCTRLA/B for 37,440 bps

; Math Shown on PDF in Pseudocode, derived for BSCALE = -7, BSEL = 299

; BAUDCTRLA = BSEL[7:0]

; BAUDCTRLB = BSCALE[3:0] : BSEL[11:8]

; BSCALE = 1001 = -7 in 2's Comp

; BSEL = 256 + 32 + 8 + 2 + 1 = 2^8 + 2^5 + 2^3 + 2^1 + 2^0

; BAUDCTRLA = 00101011

; BAUDCTRLB = 10010001

ldi r16, low(B_SEL)

sts USARTC0_BAUDCTRLA, r16

ldi r16, ((B_SCALE << 4) | high(B_SEL))

sts USARTC0_BAUDCTRLB, r16

; Set the frame format / mode in CTRLC

; In NON-SPI:

; CTRLC = CMODE[1:0] : PMODE[1:0] : SBMODE : CHSIZE[2:0]

; CMODE = 00 for Asynchronous

; PMODE = 10 for Even Parity

; SBMODE = 0 for 1 Stop Bit

; CHSIZE = 011 for 8-bit data

ldi r16, (USART_CMODE_ASYNCHRONOUS_gc | USART_PMODE_EVEN_gc | USART_CHSIZE_8BIT_gc)

sts USARTC0_CTRLC, r16

; Enable the transmit/receive in CTRLB (CTRLB[4:3]) 4 = RXEN, 3 = TXEN

lds r16, USARTC0_CTRLB

ori r16, 0b00001000 ; Enable Rx/Tx without changing previous defaults

sts USARTC0_CTRLB, r16

; Set up Interrupts in CTRLA (NOT UTILIZED HERE)

pop r16

ret

OUT_CHAR:

push r16

WAIT:

; We check the STATUS register for DREIF, which tells us the current

; transmission is done when the flag goes high. (bit 5)

lds r16, USARTC0_STATUS

sbrs r16, USART_DREIF_bp

rjmp WAIT

sts USARTC0_DATA, r17 ; Data input from main loop

pop r16

ret

Lab5_4:

/*

* Lab5_4.asm

*

*  Created: 7/4/2026 9:00:37 PM

*   Author: Evan Baesler

*/

.equ B_SCALE = -7

.equ B_SEL = 299

.org 0

rjmp MAIN

.org 0x2000

STRING_ADDR: ; String Data

.db 'H', 'i', ' ', 'H', 'o', 'w', ' ', 'A', 'r', 'e', ' ', 'Y', 'o', 'u', '?', ' ', ' ', 0

.org 0x100

MAIN:

ldi r16, low(RAMEND)

sts CPU_SPL, r16

ldi r16, high(RAMEND)

sts CPU_SPH, r16

rcall USART_INIT

ldi ZL, low(STRING_ADDR << 1)

ldi ZH, high(STRING_ADDR << 1)

LOOP:

rcall OUT_STRING

rjmp LOOP

; --SUBROUTINES-- ;

OUT_STRING:

push r16

lpm r17, Z+

; branch if null operator

tst r17

brne SKIP

ldi ZL, low(STRING_ADDR << 1)

ldi ZH, high(STRING_ADDR << 1)

SKIP:

rcall OUT_CHAR

pop r16

ret

OUT_CHAR:

push r16

WAIT:

; We check the STATUS register for DREIF, which tells us the current

; transmission is done when the flag goes high. (bit 5)

lds r16, USARTD0_STATUS

sbrs r16, USART_DREIF_bp

rjmp WAIT

sts USARTD0_DATA, r17 ; Data input from main loop

pop r16

ret

USART_INIT:

push r16

; Set the TxD pin high (PORTx_OUT) (PORTD3)

ldi r16, (1<<3)

sts PORTD_OUT, r16

; Set the TxD pin as output

sts PORTD_DIRSET, r16

; Select BSCALE and BSEL in BAUDCTRLA/B for 37,440 bps

; Math Shown on PDF in Pseudocode, derived for BSCALE = -7, BSEL = 299

; BAUDCTRLA = BSEL[7:0]

; BAUDCTRLB = BSCALE[3:0] : BSEL[11:8]

; BSCALE = 1001 = -7 in 2's Comp

; BSEL = 256 + 32 + 8 + 2 + 1 = 2^8 + 2^5 + 2^3 + 2^1 + 2^0

; BAUDCTRLA = 00101011

; BAUDCTRLB = 10010001

ldi r16, low(B_SEL)

sts USARTD0_BAUDCTRLA, r16

ldi r16, ((B_SCALE << 4) | high(B_SEL))

sts USARTD0_BAUDCTRLB, r16

; Set the frame format / mode in CTRLC

; In NON-SPI:

; CTRLC = CMODE[1:0] : PMODE[1:0] : SBMODE : CHSIZE[2:0]

; CMODE = 00 for Asynchronous

; PMODE = 10 for Even Parity

; SBMODE = 0 for 1 Stop Bit

; CHSIZE = 011 for 8-bit data

ldi r16, (USART_CMODE_ASYNCHRONOUS_gc | USART_PMODE_EVEN_gc | USART_CHSIZE_8BIT_gc)

sts USARTD0_CTRLC, r16

; Enable the transmit/receive in CTRLB (CTRLB[4:3]) 4 = RXEN, 3 = TXEN

lds r16, USARTD0_CTRLB

ori r16, 0b00001000 ; Enable Rx/Tx without changing previous defaults

sts USARTD0_CTRLB, r16

; Set up Interrupts in CTRLA (NOT UTILIZED HERE)

pop r16

ret

Lab5_5:

/*

* Lab5_5.asm

*

*  Created: 7/4/2026 9:32:48 PM

*   Author: Evan Baesler

*/

; Echo program

.equ B_SCALE = -7

.equ B_SEL = 299

.org 0

rjmp MAIN

.org 0x100

MAIN:

ldi r16, low(RAMEND)

sts CPU_SPL, r16

ldi r16, high(RAMEND)

sts CPU_SPH, r16

rcall USART_INIT

LOOP:

rcall IN_CHAR

rjmp LOOP

; --SUBROUTINES-- ;

IN_CHAR:

push r16

WAIT_IN:

; We check the STATUS register for RXCIF, which tells us the current

; transmission is done when the flag goes high. (bit 5)

lds r16, USARTD0_STATUS

sbrs r16, USART_RXCIF_bp

rjmp WAIT_IN

lds r17, USARTD0_DATA

rcall OUT_CHAR

pop r16

ret

OUT_CHAR:

push r16

WAIT_OUT:

; We check the STATUS register for DREIF, which tells us the current

; transmission is done when the flag goes high. (bit 5)

lds r16, USARTD0_STATUS

sbrs r16, USART_DREIF_bp

rjmp WAIT_OUT

sts USARTD0_DATA, r17 ; Data input from main loop

pop r16

ret

USART_INIT:

push r16

; Set the TxD/RxD pin high (PORTx_OUT) (PORTD[4:3])

ldi r16, 0b00011000

sts PORTD_OUT, r16

; Set the TxD pin as output

ldi r16, (1<<3)

sts PORTD_DIRSET, r16

ldi r16, (1<<4)

sts PORTD_DIRCLR, r16

; Select BSCALE and BSEL in BAUDCTRLA/B for 37,440 bps

; Math Shown on PDF in Pseudocode, derived for BSCALE = -7, BSEL = 299

; BAUDCTRLA = BSEL[7:0]

; BAUDCTRLB = BSCALE[3:0] : BSEL[11:8]

; BSCALE = 1001 = -7 in 2's Comp

; BSEL = 256 + 32 + 8 + 2 + 1 = 2^8 + 2^5 + 2^3 + 2^1 + 2^0

; BAUDCTRLA = 00101011

; BAUDCTRLB = 10010001

ldi r16, low(B_SEL)

sts USARTD0_BAUDCTRLA, r16

ldi r16, ((B_SCALE << 4) | high(B_SEL))

sts USARTD0_BAUDCTRLB, r16

; Set the frame format / mode in CTRLC

; In NON-SPI:

; CTRLC = CMODE[1:0] : PMODE[1:0] : SBMODE : CHSIZE[2:0]

; CMODE = 00 for Asynchronous

; PMODE = 10 for Even Parity

; SBMODE = 0 for 1 Stop Bit

; CHSIZE = 011 for 8-bit data

ldi r16, (USART_CMODE_ASYNCHRONOUS_gc | USART_PMODE_EVEN_gc | USART_CHSIZE_8BIT_gc)

sts USARTD0_CTRLC, r16

; Enable the transmit/receive in CTRLB (CTRLB[4:3]) 4 = RXEN, 3 = TXEN

lds r16, USARTD0_CTRLB

ori r16, 0b00011000 ; Enable Rx/Tx without changing previous defaults

sts USARTD0_CTRLB, r16

; Set up Interrupts in CTRLA (NOT UTILIZED HERE)

pop r16

ret

Lab5_6:

/*

* Lab5_6.asm

*

*  Created: 7/5/2026 4:00:29 PM

*   Author: Evan Baesler

*/

; Move over subroutines from Lab5_5, with modified IN_CHAR shown below and new IN_STRING

.equ STR_SIZE = 0x100

.equ B_SEL = 299

.equ B_SCALE = -7

.org 0

Rjmp MAIN

.dseg

.org 0x2000

STR_STORAGE: .byte STR_SIZE

.cseg

.org 0x100

MAIN:

ldi r16, low(RAMEND)

sts CPU_SPL, r16

ldi r16, high(RAMEND)

sts CPU_SPH, r16

rcall USART_INIT

LOOP:

; Reset Y to start of buffer before input

ldi YL, low(STR_STORAGE)

ldi YH, high(STR_STORAGE)

rcall IN_STRING

; Now echo the string

ldi YL, low(STR_STORAGE)

ldi YH, high(STR_STORAGE)

rcall OUT_STRING

; SUBROUTINES

IN_STRING:

push r16

push YL

push YH

LOOP_INSTR:

rcall IN_CHAR ; loads r17 with input character

; Check for Carriage Return, if true branch to DONE_STRING

cpi r17, 0x0D

breq DONE_STRING

; Check for Backspace, if true branch to BACK_SPACE

cpi r17, 0x08

breq BACK_SPACE

cpi r17, 0x7F

breq BACK_SPACE

; Store r17 to y, post-increment

st Y+, r17

rcall OUT_CHAR

rjmp LOOP_INSTR ; continue loop, ignore conditional statements below

BACK_SPACE:

cpi YL, low(STR_STORAGE)

brne BACK_SPACE_OK

cpi YH, high(STR_STORAGE)

breq LOOP_INSTR

BACK_SPACE_OK:

sbiw Y, 1 ; Push pointer back & replace with ' '

ldi r17, 0x08

rcall OUT_CHAR

ldi r17, ' '

rcall OUT_CHAR

ldi r17, 0x08

rcall OUT_CHAR

rjmp LOOP_INSTR

DONE_STRING:

ldi r17, 0x00

st Y, r17

pop YH

pop YL

pop r16

ret

; --------------------------------------------

OUT_STRING:

push r16

ldi r17, 0x0D

rcall OUT_CHAR

ldi YL, low(STR_STORAGE)

ldi YH, high(STR_STORAGE)

LOOP_OUT:

ld r17, Y+

tst r17

breq DONE_OUT

rcall OUT_CHAR

rjmp LOOP_OUT

DONE_OUT:

ldi r17, 0x0D

rcall OUT_CHAR

pop r16

ret

; --------------------------------------------

IN_CHAR:

push r16

WAIT_IN:

; We check the STATUS register for RXCIF, which tells us the current

; transmission is done when the flag goes high. (bit 5)

lds r16, USARTD0_STATUS

sbrs r16, USART_RXCIF_bp

rjmp WAIT_IN

lds r17, USARTD0_DATA

pop r16

ret

; --------------------------------------------

OUT_CHAR:

push r16

WAIT_OUT:

; We check the STATUS register for DREIF, which tells us the current

; transmission is done when the flag goes high. (bit 5)

lds r16, USARTD0_STATUS

sbrs r16, USART_DREIF_bp

rjmp WAIT_OUT

sts USARTD0_DATA, r17 ; Data input from main loop

pop r16

ret

; --------------------------------------------

USART_INIT:

push r16

; Set the TxD/RxD pin high (PORTx_OUT) (PORTD[4:3])

ldi r16, 0b00011000

sts PORTD_OUT, r16

; Set the TxD pin as output

ldi r16, (1<<3)

sts PORTD_DIRSET, r16

ldi r16, (1<<4)

sts PORTD_DIRCLR, r16

; Select BSCALE and BSEL in BAUDCTRLA/B for 37,440 bps

; Math Shown on PDF in Pseudocode, derived for BSCALE = -7, BSEL = 299

; BAUDCTRLA = BSEL[7:0]

; BAUDCTRLB = BSCALE[3:0] : BSEL[11:8]

; BSCALE = 1001 = -7 in 2's Comp

; BSEL = 256 + 32 + 8 + 2 + 1 = 2^8 + 2^5 + 2^3 + 2^1 + 2^0

; BAUDCTRLA = 00101011

; BAUDCTRLB = 10010001

ldi r16, low(B_SEL)

sts USARTD0_BAUDCTRLA, r16

ldi r16, ((B_SCALE << 4) | high(B_SEL))

sts USARTD0_BAUDCTRLB, r16

; Set the frame format / mode in CTRLC

; In NON-SPI:

; CTRLC = CMODE[1:0] : PMODE[1:0] : SBMODE : CHSIZE[2:0]

; CMODE = 00 for Asynchronous

; PMODE = 10 for Even Parity

; SBMODE = 0 for 1 Stop Bit

; CHSIZE = 011 for 8-bit data

ldi r16, (USART_CMODE_ASYNCHRONOUS_gc | USART_PMODE_EVEN_gc | USART_CHSIZE_8BIT_gc)

sts USARTD0_CTRLC, r16

; Enable the transmit/receive in CTRLB (CTRLB[4:3]) 4 = RXEN, 3 = TXEN

lds r16, USARTD0_CTRLB

ori r16, 0b00011000 ; Enable Rx/Tx without changing previous defaults

sts USARTD0_CTRLB, r16

; Set up Interrupts in CTRLA (NOT UTILIZED HERE)

pop r16

ret

Lab5_7:

/*

* Lab5_7.asm

*

*  Created: 7/5/2026 7:44:43 PM

*   Author: Evan Baesler

*/

.equ B_SEL = 299

.equ B_SCALE = -7

; Use USART initialization from previous sections

.org 0

rjmp MAIN

.org USARTD0_RXC_vect

rjmp INTERRUPT

MAIN:

ldi r16, (1<<6) ; PD6 (BLUE_PWM) as an output

sts PORTD_DIRSET, r16

ldi r16, low(RAMEND)

sts CPU_SPL, r16

ldi r16, high(RAMEND)

sts CPU_SPH, r16

rcall USART_INIT

rcall TC_INIT

rcall PMIC_INIT

sei

LOOP:

lds r16, TCC0_INTFLAGS ; Read the polling timer interrupt flags

sbrs r16, 0 ; Skip the toggle if the OVF (bit 0) is NOT set

rjmp LOOP ; If flag is 0, keep looping

ldi r16, (1<<6)

sts PORTD_OUTTGL, r16

ldi r16, 1 ; clear OVFIF by writing 1 to it

sts TCC0_INTFLAGS, r16

rjmp LOOP

; SUBROUTINES & INTERRUPTS

PMIC_INIT:

push r16

ldi r16, PMIC_LOLVLEN_bm

sts PMIC_CTRL, r16

Pop r16

Ret

USART_INIT:

push r16

; Set the TxD/RxD pin high (PORTx_OUT) (PORTD[4:3])

ldi r16, 0b00011000

sts PORTD_OUT, r16

; Set the TxD pin as output

ldi r16, (1<<3)

sts PORTD_DIRSET, r16

ldi r16, (1<<4)

sts PORTD_DIRCLR, r16

; Select BSCALE and BSEL in BAUDCTRLA/B for 37,440 bps

; Math Shown on PDF in Pseudocode, derived for BSCALE = -7, BSEL = 299

; BAUDCTRLA = BSEL[7:0]

; BAUDCTRLB = BSCALE[3:0] : BSEL[11:8]

; BSCALE = 1001 = -7 in 2's Comp

; BSEL = 256 + 32 + 8 + 2 + 1 = 2^8 + 2^5 + 2^3 + 2^1 + 2^0

; BAUDCTRLA = 00101011

; BAUDCTRLB = 10010001

ldi r16, low(B_SEL)

sts USARTD0_BAUDCTRLA, r16

ldi r16, ((B_SCALE << 4) | high(B_SEL))

sts USARTD0_BAUDCTRLB, r16

; Set the frame format / mode in CTRLC

; In NON-SPI:

; CTRLC = CMODE[1:0] : PMODE[1:0] : SBMODE : CHSIZE[2:0]

; CMODE = 00 for Asynchronous

; PMODE = 10 for Even Parity

; SBMODE = 0 for 1 Stop Bit

; CHSIZE = 011 for 8-bit data

ldi r16, (USART_CMODE_ASYNCHRONOUS_gc | USART_PMODE_EVEN_gc | USART_CHSIZE_8BIT_gc)

sts USARTD0_CTRLC, r16

; Enable the transmit/receive in CTRLB (CTRLB[4:3]) 4 = RXEN, 3 = TXEN

lds r16, USARTD0_CTRLB

ori r16, 0b00011000 ; Enable Rx/Tx without changing previous defaults

sts USARTD0_CTRLB, r16

; Set up Interrupts in CTRLA (NOT UTILIZED HERE)

ldi r16, USART_RXCINTLVL_LO_gc

sts USARTD0_CTRLA, r16

pop r16

ret

OUT_CHAR:

push r16

WAIT_OUT:

; We check the STATUS register for DREIF, which tells us the current

; transmission is done when the flag goes high. (bit 5)

lds r16, USARTD0_STATUS

sbrs r16, USART_DREIF_bp

rjmp WAIT_OUT

sts USARTD0_DATA, r17 ; Data input from main loop

pop r16

ret

TC_INIT:

push r16

; Set the Timer Period to 31,250 cycles for a prescaler of 64, for 1s period

ldi r16, low(31250)

sts TCC0_PER, r16

ldi r16, high(31250)

sts TCC0_PER+1, r16

ldi r16, TC_CLKSEL_DIV64_gc

sts TCC0_CTRLA, r16

pop r16

ret

INTERRUPT:

push r16

lds r16, CPU_SREG

push r16

lds r17, USARTD0_DATA ; read directly rather than waiting

; to prevent holding up loop

rcall OUT_CHAR

pop r16

sts CPU_SREG, r16

pop r16

reti

APPENDIX

`

Figure 1: Bit Transmission Measurement (Lab5_3)

Figure 2: Character Transmission Measurement (Lab5_3)

Figure 3: Program Memory Addressing

Figure 4: Data Memory Addressing

Figure 5: Ports Register Summary

Figure 6: Timer/Counter Register Summary

Figure 7: Interrupts Register Summary

Figure 8: EBI Register Summary

Figure 9: EBI Chip Select Register Summary

Figure 10: PORTH Alternate Functions

Figure 11: PORTJ Alternate Functions

Figure 12: PORTK Alternate Functions

Figure 13: USART Register Summaries

Figure 14: PORTC Alternate Functions