Evan Baesler
October 4, 2026

31151619

EE Design 1 Technical Report

Microcontroller ADC & LCD

Introduction

In this module, our goal was to create an ohmmeter through a Raspberry Pi Pico 2 and an LCD. To do this we had to use our ADCs and a voltage divider with one known resistor and one unknown resistor that we would use to test. The margins for this design were to be within 5% of the resistance of any resistance between 1kOhm and 1MOhm.

Methods

The first step in designing our ohmmeter was to define the hardware scope. For this project, we knew we would need an LCD, Raspberry Pi Pico 2, potentiometers for brightness and contrast, and a voltage divider to determine the resistance of the unknown resistor.

Figure 1: Hardware (Left) and Software (Right) Flowchart / Functional Block Diagram.

The resistance of our unknown resistor is equivalent to:

Rx=Rk*VxVin-Vx

Which gives us the insight that we can apply that formula to our ADC to derive the value of Rx. While the fundamental design is elementary on a surface level, there is a lot of nuance to the implementation. For example, some calibration issues we run into are:

- Power supply and regulation limitations of the Raspberry Pi Pico 2

- Electrical Noise

- Accuracy of known resistors

- ADC quantization error and linearity

From most severe to least, we first notice a large drop in the power supply of the

Raspberry Pi Pico 2, which can quickly cause our rails to fluctuate, which can massively affect the program when hard-coded for an expected rail of 3.3V; so we use an exterior power supply with higher current capacity.

Then, we see electrical noise on the ADCs & rail, so we apply 100nF capacitors to bypass any electrical noise straight to ground. This only requires two capacitors as ADC_Vin is shorted to VCC, meaning that the capacitor there shorts both VCC and ADC_Vin to GND; while ADC_Vx requires a separate bypass.

Along with this, we find that the Rk must be inspected for its true value, as there are tolerances from the manufacturer that must be accounted for. In this model, the exact resistance found was 37580 Ohms on a 37K.

For the ADC, the first thing we find is ADC input noise is a major factor of our design (1), as there is kT/C resistor noise, and there are limitations on the amount of bits that can be free of noise. This works with and against us in some instances, as some noise is good as it allows the results to ever-so-slightly fluctuate as to allow for averages between two values to happen, otherwise; the results, if slightly offset, would remain fixed with that offset. We also find that a 12-bit ADC can represent 4096 values, which means that our minimum voltage step is 3.3/4096 = 0.00080566406V, which led to limitations on the higher end of our spectrum. This, combined with a high input impedance on the ADC when we reach higher loads meant that higher Rx values led to unreliable outputs versus expectations.

To compensate, resistors of known sizes were placed on the board after calibration and a piecewise function was used to tune the values read to the actual values, which significantly brought higher accuracy. We also calibrate the machine when first turned on to find the open circuit offset and bring the resistance to 0.

Figure 2: Altium Schematic of the Raspberry Pi Pico 2 LCD Ohmmeter

Results

Figure 3: Picture of Ohmmeter in Action

As mentioned above, to calibrate our ohmmeter, we utilized an open circuit offset, a piecewise function, bypass capacitors, external voltage regulation and power supply.

Conclusion

This module reinforced our design capabilities by making us think from the ground up about making something as commonly used as an ohmmeter and showed the full hardware to software to implementation pipeline and how to refine on the go. This module also helped us realize the actual implementation of commercial grade oscilloscopes, with multiple comparison resistors for different ranges, noise compensation, powerful supplies, and internal transistor circuitry to automate the switching processes.

Appendix

# ---------------------------------

#   CFAH1602Z-YYH-ET LCD Firmware

# ---------------------------------

# Written by Evan Baesler

# September 26, 2026

# For: Raspberry Pi Pico 2

from machine import ADC, Pin

import utime

# Pins are in terms of GPIO Ports

# RW, D0-D3 to GND

# LEGEND:

# XP = External Potentiometer Control

# Contrast Control (XP)

# Backlight Brightness (XP)

# Chip Enable

EN = Pin(6, Pin.OUT) # Enable to push data into LCD

# Register Select

RS = Pin(5, Pin.OUT) # 0 = command | 1 = data

# Data bus (lower bits are not utilized in 4-bit mode!)

D4 = Pin(10, Pin.OUT)

D5 = Pin(11, Pin.OUT)

D6 = Pin(12, Pin.OUT)

D7 = Pin(13, Pin.OUT)

# ADC Pins

adc_vx = ADC(28)

adc_vin = ADC(26)

PORT = [10, 11, 12, 13]

L = [0,0,0,0]

## SUBROUTINES

def configure():

for i in range(4):

L[i] = Pin(PORT[i], Pin.OUT)

def lcd_strobe():

EN.value(1) # Send Data to LCD

utime.sleep_ms(1) # Sleep for 1 ms

EN.value(0) # Read Command / Data

utime.sleep_ms(1) # Sleep for 1 ms

# ASCII is an 8-bit data format, so we need to do

# the upper and lower nibble individually

def lcd_write(c, mode):

# FIRST NIBBLE (upper 4 bits)

if mode == 0: # Command in numerical form

d = c

else:

d = ord(c) # Convert Hexadecimal to ASCII

d = d >> 4 # Shift right to access upper nibble

for i in range(4): # Put bits to D4-D7

b = d & 1 # Extract lowest bit

L[i].value(b) # Set that GPIO pin

d = d >> 1 # Shift right to next bit

RS.value(mode) # Set RS via mode

lcd_strobe() # Pulse EN to latch upper byte

# SECOND NIBBLE (lower 4 bits)

if mode == 0: # Command in Hexadecimal

d = c

else:

d = ord(c) # Convert Hexadecimal to ASCII

for i in range(4): # Put bits to D4-D7

b = d & 1 # Extract lowest bit

L[i].value(b) # Set that GPIO pin

d = d >> 1 # Shift right to next bit

RS.value(mode) # Set RS via mode

lcd_strobe() # Pulse EN to latch upper byte

utime.sleep_ms(1)

RS.value(1) # Return RS to data mode

def lcd_clear():

lcd_write(0x01, 0)

utime.sleep_ms(5)

def lcd_home():

lcd_write(0x02, 0)

utime.sleep_ms(5)

def lcd_cursor_blink():

lcd_write(0x0D, 0)

utime.sleep_ms(1)

def lcd_cursor_on():

lcd_write(0x0E, 0)

utime.sleep_ms(1)

def lcd_cursor_off():

lcd_write(0x0C, 0)

utime.sleep_ms(1)

def lcd_puts(s):

l = len(s)

for i in range(l):

lcd_putch(s[i])

def lcd_putch(c):

lcd_write(c, 1)

def lcd_goto(col, row):

c = col + 1

if row == 0:

address = 0

if row == 1:

address = 0x40

if row == 2:

address = 0x14

if row == 3:

address = 0x54

address = address + c - 1

lcd_write(0x80 | address, 0)

def lcd_init():

configure()

RS.value(0)

EN.value(0)

utime.sleep_ms(120)

for i in range(4):

L[i].value(0)

utime.sleep_ms(50)

L[0].value(1)

L[1].value(1)

lcd_strobe()

utime.sleep_ms(10)

lcd_strobe()

utime.sleep_ms(10)

lcd_strobe()

utime.sleep_ms(10)

L[0].value(0)

lcd_strobe()

utime.sleep_ms(5)

lcd_write(0x28, 0)

utime.sleep_ms(1)

lcd_write(0x08, 0)

utime.sleep_ms(1)

lcd_write(0x01, 0)

utime.sleep_ms(10)

lcd_write(0x06, 0)

utime.sleep_ms(5)

lcd_write(0x0C, 0)

utime.sleep_ms(10)

# ADC (returns averaged raw counts, 0-65535)

def adc_read_both(samples=100000):

vin_total = 0

vx_total = 0

for i in range(samples):

vin_total += adc_vin.read_u16()

vx_total += adc_vx.read_u16()

return vin_total / samples, vx_total / samples

def calculate_resistance(vin, vx, offset=0):

rk = 37580

if (vin - vx) > 0:

rx_unscaled = ((rk * vx) / (vin - vx)) - offset

if rx_unscaled > 6000000: # 1M

rx = rx_unscaled * 1000/7000

elif rx_unscaled > 3500000: # 900K

rx = rx_unscaled * 900/4050

elif rx_unscaled > 2500000: # 800K

rx = rx_unscaled * 800/2700

elif rx_unscaled > 1500000: # 700K

rx = rx_unscaled * 700/1850

elif rx_unscaled > 1000000: # 600K

rx = rx_unscaled * 600/1355

elif rx_unscaled > 700000: # 500K

rx = rx_unscaled * 500/960

elif rx_unscaled > 500000: # 400K

rx = rx_unscaled * 400/671

elif rx_unscaled > 290000: # 300K

rx = rx_unscaled * 300/445

elif rx_unscaled > 190000: # 200K

rx = rx_unscaled * 200/265

elif rx_unscaled > 90000: # 100K

rx = rx_unscaled * 100/120

elif rx_unscaled > 35000: # 40K

rx = rx_unscaled * 40000/45500

elif rx_unscaled > 20000: # 20K

rx = rx_unscaled * 20000/22359

elif rx_unscaled > 9000: # 10K

rx = rx_unscaled * 10000/11000

elif rx_unscaled > 900: # 1K

rx = rx_unscaled * 1000/1110

elif rx_unscaled < 400: # Too low

return None, None

else:

rx = rx_unscaled

return rx, rx_unscaled

return None, None

def adc_calibrate():

lcd_clear()

lcd_puts("Short Terminals!")

utime.sleep_ms(1000)

lcd_clear()

lcd_puts("Calibrating...")

vin, vx = adc_read_both(10000)

rx, rx_unscaled = calculate_resistance(vin, vx, 0)

lcd_clear()

return rx_unscaled if rx_unscaled is not None else 0

def lcd_display_resistance(offset):

vin, vx = adc_read_both(10000)

rx, rx_unscaled = calculate_resistance(vin, vx, offset)

lcd_clear()

if rx is None or rx > 1000000:

lcd_puts("Rx: Out of Range")

#    lcd_goto(0, 1)

#    lcd_puts("Raw: Out of Range")

else:

rx = max(0, rx)

rx_unscaled = max(0, rx_unscaled)

lcd_puts("Rx: " + str(int(rx)))

#    lcd_goto(0, 1)

#    lcd_puts("Raw: " + str(int(rx_unscaled)))

## END OF SUBROUTINES

# PROGRAM START

lcd_init()

lcd_clear()

offset = adc_calibrate()

while True:

lcd_display_resistance(offset)

utime.sleep_ms(500)

# PROGRAM END

References

1. https://www.analog.com/en/resources/analog-dialogue/articles/adc-input-noise.html

2. ChatGPT / Google Gemini