Full Name: Evan Baesler

EEL 3135	– Classwork #01

Question #1:	Consider the signal x(t) = 2cos(5t)

- Graph x(t) for −2π ≤ t ≤ 2π using MATLAB. Provide code and results.

figure(1)

t = -2*pi:pi/200:2*pi;

y = 2*cos(5*t);

plot(theta, y)

xlabel('Angle (Radians)')

ylabel('Amplitude')

Graph a shifted version x(t − 1) for the same interval using MATLAB. Provide code and results.

- Explain how the shift affects the phase of the signal.

It shifts the wave to the right by 1.

- Compare the frequency and amplitude of the two signals.

The frequency and the amplitude are the same.

Question #2:	Phasors and Complex Representation

A sinusoid is given by y(t) = 3cos(10πt + π/6).

- What is the angular frequency, conventional frequency, and phase of y(t)?

10pi, 5Hz, pi/6 phase.

- Write the phasor representation

  - = Aejϕ

Y = 3ej(π/6).

- Determine the real and imaginary parts of

  - = ejω0t.

Real: 2.598, Imaginary: 1.5

- Two phasors are defined as

z1 = 2 + j,	z2 = −1 + 3j.

Find their sum

zs = z1 + z2 = 1 + 4j

and express in both rectangular and polar form.

Rectangular: 1 + 4j, Polar: sqrt(17)phasor(75.96)

Question #3:	Simplification Using Trigonometric Identities

- Consider the sinusoid

.

Determine the amplitude A, angular frequency ω, and phase ϕ such that

x(t) = Asin(ωt + ϕ).

- Simplify the following sum into the form Acos(2πft + Φ):

4cos(20πt + π/3) + 2cos(20πt − π/3)

- Simplify the product cos(12πt)cos(6πt)