REQUIREMENTS NOT MET

None

PROBLEMS ENCOUNTERED

None Encountered

FUTURE WORK/APPLICATIONS

Finally, we wrap up this awesome semester with utilizing the DAC and DMA. Through the utilization of these two peripherals, we are able to rapidly transmit data, allowing us to modify the delivered frequency to simulate musical notes. The utilization of these peripherals could allow us to create varying frequency waveforms, potentially for wireless communications and audio creation.

Thanks for the awesome semester .

PRE-LAB EXERCISES

- Why might you be unable to generate a desired frequency with this method of using an interrupt? Refer to the disassembly of the interrupt service routine. Additionally, temporarily change the optimization level of your compiler to -O1. Are the results any different? Why or why not?

If the timer is running too quickly, the ISR may not be available within time to actually trigger as it is still executing commands behind the scenes, which would cause it to only update when the ISR is available, seemingly at a rate around 1 kHz.

Changing the optimization level to -O1 DOES fix the error; allowing for sine waves over 1kHz (around 1500 Hz maximum), probably by removing the ISR and instead polling, allowing for much faster updates.

- Would a method of synchronous polling (i.e., a method with no interrupts) result in the same issue identified in the previous exercise? In other words, would the desired frequency not initially met now be achieved? Alter your program to check your answer, and then take a screenshot of the waveform generated, again denoting a precise frequency measurement of this waveform within the screenshot.

It likely would improve the maximum frequency considerably, causing the update rate to be limited by the processor speed rather than the ISR availability.

- What is the correlation between the amount of data points used to recreate the waveform and the overall quality of the waveform?

A higher data point count means higher resolution, where we are able to visualize more individual steps along the sine wave. However, with higher quality resolution, we also have more data to process and this directly lowers our processing speed, and effectively our maximum frequencies.

PSEUDOCODE / FLOWCHARTS

Lab8_1.c

Initialize Clock @ 32MHz & DACA

Sine_table[256] = sine_table data

While 1{

Wait for DAC:

Update DAC

}

Daca_init:

Select AREFB

Select Single CH0

Enable DAC & CH0

Lab8_2a.c

Initialize Clock @ 32MHz & DACA & TCC0

Enable SEI

Infinite while loop{}

Daca_init from lab8_1.c

Tcc0_init:

Set period to 659.26 Hz

Set Prescaler

Enable lo-level interrupts

Enable PMIC for lo-level interrupts

ISR(TCC0_OVF):

If (count < 255):

Count += 1

Else:

Count = 0

DACA.DATA = sine_table[count]

Lab8_2b.c

Initialize Clock @ 32MHz & DACA & TCC0

Enable SEI

Infinite while loop:

If TCC0 OVF:

DACA.DATA = sine_table[count];

If count < 255:

Count += 1

Else:

Count = 0

Daca_init:

Select AREFB (2.5V)

Select CH0

Enable DAC & CH0

Tcc0_init:

Select period

Select prescaler

Lab8_3.c

Initialize Clock @ 32MHz & DACA & TCC0 & DMAC

Infinite while loop:

Daca_init:

Select AREFB (2.5V)

Select CH0

Enable DAC & CH0

Tcc0_init:

Select period

Select prescaler

Dmac_init:

Select infinite repeats

Address source & destination

TRFCNT = 512 (256 * 2)

Select REPEAT, SINGLE, 2BYTE for CTRLA

Select BLOCK SRC LD, INC SRCDIR, BURST DESTRELOAD, INC DESTDIR for ADDRCTLR

EVSYS fires on TCC0 OVF, DMAC triggered by EVSYS CH0

Enable DMA / CH0

Lab8_4.c

Initialize Clock @ 32MHz & DACA & TCC0 & DMAC

Enable Audio Amp

Infinite while loop:

Daca_init:

Select AREFB (2.5V)

Select CH1

Enable DAC & CH0

Tcc0_init:

Reset CNT

Reset flags

Select period

Select prescaler

Dmac_init:

Select infinite repeats

Address source & destination

TRFCNT = 512 (256 * 2)

Select REPEAT, SINGLE, 2BYTE for CTRLA

Select BLOCK SRC LD, INC SRCDIR, BURST DESTRELOAD, INC DESTDIR for ADDRCTLR

EVSYS fires on TCC0 OVF, DMAC triggered by EVSYS CH0

Enable DMA / CH0

Lab8_5.c

Initialize Clock @ 32MHz & DACA & TCC0 & TCC1 & DMAC & USARTD0

Create baud rate, period, mode variables

// TCC0 is Playback, TCC1 is Delay

Create Sine & Tri tables

Create a Pointer, set to Sine table

Enable Audio Amp

Infinite while loop:

Wait for USART:

Take input character

Compare character, set to respective note or change mode

Wait for TCC1 OVF

Reset flag

Turn off playback timer

Daca_init:

Select AREFB (2.5V)

Select CH1

Enable DAC & CH0

Tcc0_init:

Reset CNT

Reset flags

Select period

Select prescaler

Tcc1_init:

Reset CNT

Set period

Reset flag

Turn off

Dmac_init:

Select infinite repeats

Address source & destination

TRFCNT = 512 (256 * 2)

Select REPEAT, SINGLE, 2BYTE for CTRLA

Select BLOCK SRC LD, INC SRCDIR, BURST DESTRELOAD, INC DESTDIR for ADDRCTLR

EVSYS fires on TCC0 OVF, DMAC triggered by EVSYS CH0

Enable DMA / CH0

Usartd0_init:

Set TX/RX pins

Set baud rate

Set PuTTy configs & enable

Usartd0_in:

Wait for USART:

Return char

Set_wave(mode):

Disable DMA

Check mode:

Select mode via pointer

Update addressing

Reset TRFCNT

Enable DMA

Set_note(freq):

Update Tcc0.PER with freq

Reset Tcc0.CNT

Enable prescaler

Reset Tcc1.CNT

Reset Tcc1 flags

Enable Tcc1 prescaler

Note_timer:

Reset Tcc1.CT

Reset Tcc1 flags

Enable timer

PROGRAM CODE

Lab8_1.c

/*

* lab8_1.c

*

* Created: 7/21/2026 9:25:58 PM

* Author : Evan Baesler

*/

#include <avr/io.h>

extern void clock_init(void);

void daca_init(void);

uint16_t value = 2621;

int main(void)

{

clock_init(); // Sets Clock to 32MHz

daca_init();

while (1)

{

// Poll until DAC channel 0 data register is empty

if (DACA.STATUS & DAC_CH0DRE_bm)

{

DACA.CH0DATA = value;

}

}

}

void daca_init(void){

DACA.CTRLC = DAC_REFSEL_AREFB_gc; // REFSEL[1:0] on bits 4:3, AREFB = 2.5V

DACA.CTRLB = DAC_CHSEL_SINGLE_gc;

DACA.CTRLA = DAC_ENABLE_bm | DAC_CH0EN_bm;

}

Lab8_2a.c

/*

* lab8_2.c

*

* Created: 7/26/2026 10:15:16 PM

*  Author: Evan Baesler

*/

#include <avr/io.h>

#include <avr/interrupt.h>

extern void clock_init(void);

void daca_init(void);

void tcc0_init(void);

uint16_t count = 0;

// 256 Value Sine Wave Approximation

uint16_t sine_table[256] =

{0x800, 0x832, 0x864, 0x896, 0x8c8, 0x8fa, 0x92c, 0x95e,

0x98f, 0x9c0, 0x9f1, 0xa22, 0xa52, 0xa82, 0xab1, 0xae0,

0xb0f, 0xb3d, 0xb6b, 0xb98, 0xbc5, 0xbf1, 0xc1c, 0xc47,

0xc71, 0xc9a, 0xcc3, 0xceb, 0xd12, 0xd39, 0xd5f, 0xd83,

0xda7, 0xdca, 0xded, 0xe0e, 0xe2e, 0xe4e, 0xe6c, 0xe8a,

0xea6, 0xec1, 0xedc, 0xef5, 0xf0d, 0xf24, 0xf3a, 0xf4f,

0xf63, 0xf76, 0xf87, 0xf98, 0xfa7, 0xfb5, 0xfc2, 0xfcd,

0xfd8, 0xfe1, 0xfe9, 0xff0, 0xff5, 0xff9, 0xffd, 0xffe,

0xfff, 0xffe, 0xffd, 0xff9, 0xff5, 0xff0, 0xfe9, 0xfe1,

0xfd8, 0xfcd, 0xfc2, 0xfb5, 0xfa7, 0xf98, 0xf87, 0xf76,

0xf63, 0xf4f, 0xf3a, 0xf24, 0xf0d, 0xef5, 0xedc, 0xec1,

0xea6, 0xe8a, 0xe6c, 0xe4e, 0xe2e, 0xe0e, 0xded, 0xdca,

0xda7, 0xd83, 0xd5f, 0xd39, 0xd12, 0xceb, 0xcc3, 0xc9a,

0xc71, 0xc47, 0xc1c, 0xbf1, 0xbc5, 0xb98, 0xb6b, 0xb3d,

0xb0f, 0xae0, 0xab1, 0xa82, 0xa52, 0xa22, 0x9f1, 0x9c0,

0x98f, 0x95e, 0x92c, 0x8fa, 0x8c8, 0x896, 0x864, 0x832,

0x800, 0x7cd, 0x79b, 0x769, 0x737, 0x705, 0x6d3, 0x6a1,

0x670, 0x63f, 0x60e, 0x5dd, 0x5ad, 0x57d, 0x54e, 0x51f,

0x4f0, 0x4c2, 0x494, 0x467, 0x43a, 0x40e, 0x3e3, 0x3b8,

0x38e, 0x365, 0x33c, 0x314, 0x2ed, 0x2c6, 0x2a0, 0x27c,

0x258, 0x235, 0x212, 0x1f1, 0x1d1, 0x1b1, 0x193, 0x175,

0x159, 0x13e, 0x123, 0x10a, 0xf2, 0xdb, 0xc5, 0xb0,

0x9c, 0x89, 0x78, 0x67, 0x58, 0x4a, 0x3d, 0x32,

0x27, 0x1e, 0x16, 0xf, 0xa, 0x6, 0x2, 0x1,

0x0, 0x1, 0x2, 0x6, 0xa, 0xf, 0x16, 0x1e,

0x27, 0x32, 0x3d, 0x4a, 0x58, 0x67, 0x78, 0x89,

0x9c, 0xb0, 0xc5, 0xdb, 0xf2, 0x10a, 0x123, 0x13e,

0x159, 0x175, 0x193, 0x1b1, 0x1d1, 0x1f1, 0x212, 0x235,

0x258, 0x27c, 0x2a0, 0x2c6, 0x2ed, 0x314, 0x33c, 0x365,

0x38e, 0x3b8, 0x3e3, 0x40e, 0x43a, 0x467, 0x494, 0x4c2,

0x4f0, 0x51f, 0x54e, 0x57d, 0x5ad, 0x5dd, 0x60e, 0x63f,

0x670, 0x6a1, 0x6d3, 0x705, 0x737, 0x769, 0x79b, 0x7cd};

int main(void)

{

clock_init(); // Sets Clock to 32MHz

daca_init();

tcc0_init();

sei();

while (1)

{

// Poll until DAC channel 0 data register is empty

}

}

void daca_init(void){

DACA.CTRLC = DAC_REFSEL_AREFB_gc; // REFSEL[1:0] on bits 4:3, AREFB = 2.5V

DACA.CTRLB = DAC_CHSEL_SINGLE_gc;

DACA.CTRLA = DAC_ENABLE_bm | DAC_CH0EN_bm;

}

void tcc0_init(void){

TCC0.PER = 192; // 1/659.26(32MHz/1) = 48539/256 = 190, adjusted to 192 for accuracy based on clock speed variance

TCC0.CTRLA = TC_CLKSEL_DIV1_gc;

TCC0.INTCTRLA = TC_OVFINTLVL_LO_gc;

PMIC.CTRL = PMIC_LOLVLEN_bm;

}

ISR(TCC0_OVF_vect){

if (count < 255){

count += 1;

}

else {

count = 0;

}

DACA.CH0DATA = sine_table[count];

}

Lab8_2b.c

/*

* lab8_2.c

*

* Created: 7/27/2026 10:58:55 PM

*  Author: Evan Baesler

*/

#include <avr/io.h>

#include <avr/interrupt.h>

extern void clock_init(void);

void daca_init(void);

void tcc0_init(void);

uint16_t count = 0;

// 256 Value Sine Wave Approximation

uint16_t sine_table[256] =

{0x800, 0x832, 0x864, 0x896, 0x8c8, 0x8fa, 0x92c, 0x95e,

0x98f, 0x9c0, 0x9f1, 0xa22, 0xa52, 0xa82, 0xab1, 0xae0,

0xb0f, 0xb3d, 0xb6b, 0xb98, 0xbc5, 0xbf1, 0xc1c, 0xc47,

0xc71, 0xc9a, 0xcc3, 0xceb, 0xd12, 0xd39, 0xd5f, 0xd83,

0xda7, 0xdca, 0xded, 0xe0e, 0xe2e, 0xe4e, 0xe6c, 0xe8a,

0xea6, 0xec1, 0xedc, 0xef5, 0xf0d, 0xf24, 0xf3a, 0xf4f,

0xf63, 0xf76, 0xf87, 0xf98, 0xfa7, 0xfb5, 0xfc2, 0xfcd,

0xfd8, 0xfe1, 0xfe9, 0xff0, 0xff5, 0xff9, 0xffd, 0xffe,

0xfff, 0xffe, 0xffd, 0xff9, 0xff5, 0xff0, 0xfe9, 0xfe1,

0xfd8, 0xfcd, 0xfc2, 0xfb5, 0xfa7, 0xf98, 0xf87, 0xf76,

0xf63, 0xf4f, 0xf3a, 0xf24, 0xf0d, 0xef5, 0xedc, 0xec1,

0xea6, 0xe8a, 0xe6c, 0xe4e, 0xe2e, 0xe0e, 0xded, 0xdca,

0xda7, 0xd83, 0xd5f, 0xd39, 0xd12, 0xceb, 0xcc3, 0xc9a,

0xc71, 0xc47, 0xc1c, 0xbf1, 0xbc5, 0xb98, 0xb6b, 0xb3d,

0xb0f, 0xae0, 0xab1, 0xa82, 0xa52, 0xa22, 0x9f1, 0x9c0,

0x98f, 0x95e, 0x92c, 0x8fa, 0x8c8, 0x896, 0x864, 0x832,

0x800, 0x7cd, 0x79b, 0x769, 0x737, 0x705, 0x6d3, 0x6a1,

0x670, 0x63f, 0x60e, 0x5dd, 0x5ad, 0x57d, 0x54e, 0x51f,

0x4f0, 0x4c2, 0x494, 0x467, 0x43a, 0x40e, 0x3e3, 0x3b8,

0x38e, 0x365, 0x33c, 0x314, 0x2ed, 0x2c6, 0x2a0, 0x27c,

0x258, 0x235, 0x212, 0x1f1, 0x1d1, 0x1b1, 0x193, 0x175,

0x159, 0x13e, 0x123, 0x10a, 0xf2, 0xdb, 0xc5, 0xb0,

0x9c, 0x89, 0x78, 0x67, 0x58, 0x4a, 0x3d, 0x32,

0x27, 0x1e, 0x16, 0xf, 0xa, 0x6, 0x2, 0x1,

0x0, 0x1, 0x2, 0x6, 0xa, 0xf, 0x16, 0x1e,

0x27, 0x32, 0x3d, 0x4a, 0x58, 0x67, 0x78, 0x89,

0x9c, 0xb0, 0xc5, 0xdb, 0xf2, 0x10a, 0x123, 0x13e,

0x159, 0x175, 0x193, 0x1b1, 0x1d1, 0x1f1, 0x212, 0x235,

0x258, 0x27c, 0x2a0, 0x2c6, 0x2ed, 0x314, 0x33c, 0x365,

0x38e, 0x3b8, 0x3e3, 0x40e, 0x43a, 0x467, 0x494, 0x4c2,

0x4f0, 0x51f, 0x54e, 0x57d, 0x5ad, 0x5dd, 0x60e, 0x63f,

0x670, 0x6a1, 0x6d3, 0x705, 0x737, 0x769, 0x79b, 0x7cd};

int main(void)

{

clock_init(); // Sets Clock to 32MHz

daca_init();

tcc0_init();

sei();

while (1)

{

if (TCC0.INTFLAGS & TC0_OVFIF_bm){

TCC0.INTFLAGS = TC0_OVFIF_bm;

DACA.CH0DATA = sine_table[count];

if (count < 255){

count += 1;

}

else {

count = 0;

}

}

}

}

void daca_init(void){

DACA.CTRLC = DAC_REFSEL_AREFB_gc; // REFSEL[1:0] on bits 4:3, AREFB = 2.5V

DACA.CTRLB = DAC_CHSEL_SINGLE_gc;

DACA.CTRLA = DAC_ENABLE_bm | DAC_CH0EN_bm;

}

void tcc0_init(void){

TCC0.PER = 85; // 1/659.26(32MHz/1) = 84, adjusted to 85 for accuracy based on clock speed variance

TCC0.CTRLA = TC_CLKSEL_DIV1_gc;

}

Lab8_3.c

/*

* lab8_3.c

*

* Created: 7/27/2026 11:31:17 PM

*  Author: Evan Baesler

*/

#include <avr/io.h>

extern void clock_init(void);

void daca_init(void);

void tcc0_init(void);

void dmac_init(void);

// 256 Value Sine Wave Approximation

uint16_t sine_table[256] =

{0x800, 0x832, 0x864, 0x896, 0x8c8, 0x8fa, 0x92c, 0x95e,

0x98f, 0x9c0, 0x9f1, 0xa22, 0xa52, 0xa82, 0xab1, 0xae0,

0xb0f, 0xb3d, 0xb6b, 0xb98, 0xbc5, 0xbf1, 0xc1c, 0xc47,

0xc71, 0xc9a, 0xcc3, 0xceb, 0xd12, 0xd39, 0xd5f, 0xd83,

0xda7, 0xdca, 0xded, 0xe0e, 0xe2e, 0xe4e, 0xe6c, 0xe8a,

0xea6, 0xec1, 0xedc, 0xef5, 0xf0d, 0xf24, 0xf3a, 0xf4f,

0xf63, 0xf76, 0xf87, 0xf98, 0xfa7, 0xfb5, 0xfc2, 0xfcd,

0xfd8, 0xfe1, 0xfe9, 0xff0, 0xff5, 0xff9, 0xffd, 0xffe,

0xfff, 0xffe, 0xffd, 0xff9, 0xff5, 0xff0, 0xfe9, 0xfe1,

0xfd8, 0xfcd, 0xfc2, 0xfb5, 0xfa7, 0xf98, 0xf87, 0xf76,

0xf63, 0xf4f, 0xf3a, 0xf24, 0xf0d, 0xef5, 0xedc, 0xec1,

0xea6, 0xe8a, 0xe6c, 0xe4e, 0xe2e, 0xe0e, 0xded, 0xdca,

0xda7, 0xd83, 0xd5f, 0xd39, 0xd12, 0xceb, 0xcc3, 0xc9a,

0xc71, 0xc47, 0xc1c, 0xbf1, 0xbc5, 0xb98, 0xb6b, 0xb3d,

0xb0f, 0xae0, 0xab1, 0xa82, 0xa52, 0xa22, 0x9f1, 0x9c0,

0x98f, 0x95e, 0x92c, 0x8fa, 0x8c8, 0x896, 0x864, 0x832,

0x800, 0x7cd, 0x79b, 0x769, 0x737, 0x705, 0x6d3, 0x6a1,

0x670, 0x63f, 0x60e, 0x5dd, 0x5ad, 0x57d, 0x54e, 0x51f,

0x4f0, 0x4c2, 0x494, 0x467, 0x43a, 0x40e, 0x3e3, 0x3b8,

0x38e, 0x365, 0x33c, 0x314, 0x2ed, 0x2c6, 0x2a0, 0x27c,

0x258, 0x235, 0x212, 0x1f1, 0x1d1, 0x1b1, 0x193, 0x175,

0x159, 0x13e, 0x123, 0x10a, 0xf2, 0xdb, 0xc5, 0xb0,

0x9c, 0x89, 0x78, 0x67, 0x58, 0x4a, 0x3d, 0x32,

0x27, 0x1e, 0x16, 0xf, 0xa, 0x6, 0x2, 0x1,

0x0, 0x1, 0x2, 0x6, 0xa, 0xf, 0x16, 0x1e,

0x27, 0x32, 0x3d, 0x4a, 0x58, 0x67, 0x78, 0x89,

0x9c, 0xb0, 0xc5, 0xdb, 0xf2, 0x10a, 0x123, 0x13e,

0x159, 0x175, 0x193, 0x1b1, 0x1d1, 0x1f1, 0x212, 0x235,

0x258, 0x27c, 0x2a0, 0x2c6, 0x2ed, 0x314, 0x33c, 0x365,

0x38e, 0x3b8, 0x3e3, 0x40e, 0x43a, 0x467, 0x494, 0x4c2,

0x4f0, 0x51f, 0x54e, 0x57d, 0x5ad, 0x5dd, 0x60e, 0x63f,

0x670, 0x6a1, 0x6d3, 0x705, 0x737, 0x769, 0x79b, 0x7cd};

int main(void)

{

clock_init(); // Sets Clock to 32MHz

daca_init();

tcc0_init();

dmac_init();

while (1)

{

// NO OP

}

}

void daca_init(void){

DACA.CTRLC = DAC_REFSEL_AREFB_gc; // REFSEL[1:0] on bits 4:3, AREFB = 2.5V

DACA.CTRLB = DAC_CHSEL_SINGLE_gc;

DACA.CTRLA = DAC_ENABLE_bm | DAC_CH0EN_bm;

}

void tcc0_init(void){

TCC0.PER = 85; // 32MHz/(256*1479.98) ~= 84.5, adjusted for accuracy

TCC0.CTRLA = TC_CLKSEL_DIV1_gc;

}

void dmac_init(void){

DMA.CH0.REPCNT = 0; // Infinite Repeats

// Source Addressing thru pointers

DMA.CH0.SRCADDR0= (uint8_t)((uintptr_t) sine_table);

DMA.CH0.SRCADDR1= (uint8_t)(((uintptr_t) sine_table) >> 8);

DMA.CH0.SRCADDR2= (uint8_t)(((uint32_t)((uintptr_t)sine_table)) >>16);

// Destination Addressing

DMA.CH0.DESTADDR0= (uint8_t)((uintptr_t) &DACA.CH0DATA);

DMA.CH0.DESTADDR1= (uint8_t)(((uintptr_t) &DACA.CH0DATA) >> 8);

DMA.CH0.DESTADDR2= (uint8_t)(((uint32_t)((uintptr_t)&DACA.CH0DATA)) >>16);

DMA.CH0.TRFCNT = 512; // 256 samples * 2 bytes

// Turn on repeat, CH0, and 2BYTE burst

DMA.CH0.CTRLA = DMA_CH_REPEAT_bm | DMA_CH_SINGLE_bm | DMA_CH_BURSTLEN_2BYTE_gc; // one 2-byte sample per trigger

// Reload full block, increment source/output, reload in bursts

DMA.CH0.ADDRCTRL =

DMA_CH_SRCRELOAD_BLOCK_gc |

DMA_CH_SRCDIR_INC_gc |

DMA_CH_DESTRELOAD_BURST_gc |

DMA_CH_DESTDIR_INC_gc;

// Timer overflow -> Event Channel 0 -> DMA Update

EVSYS.CH0MUX = EVSYS_CHMUX_TCC0_OVF_gc;

DMA.CH0.TRIGSRC = DMA_CH_TRIGSRC_EVSYS_CH0_gc;

DMA.CH0.CTRLA |= DMA_CH_ENABLE_bm;

DMA.CTRL = DMA_ENABLE_bm; // Activate DMA

}

Lab8_4.c

/*

* lab8_4.c

*

* Created: 7/28/2026 9:56:28 PM

*   Author: Evan Baesler

*/

#include <avr/io.h>

// PROTOTYPES

extern void clock_init(void);

void daca_init(void);

void tcc0_init(void);

void dmac_init(void);

// 256 Value Sine Wave Approximation

uint16_t sine_table[256] =

{0x800, 0x832, 0x864, 0x896, 0x8c8, 0x8fa, 0x92c, 0x95e,

0x98f, 0x9c0, 0x9f1, 0xa22, 0xa52, 0xa82, 0xab1, 0xae0,

0xb0f, 0xb3d, 0xb6b, 0xb98, 0xbc5, 0xbf1, 0xc1c, 0xc47,

0xc71, 0xc9a, 0xcc3, 0xceb, 0xd12, 0xd39, 0xd5f, 0xd83,

0xda7, 0xdca, 0xded, 0xe0e, 0xe2e, 0xe4e, 0xe6c, 0xe8a,

0xea6, 0xec1, 0xedc, 0xef5, 0xf0d, 0xf24, 0xf3a, 0xf4f,

0xf63, 0xf76, 0xf87, 0xf98, 0xfa7, 0xfb5, 0xfc2, 0xfcd,

0xfd8, 0xfe1, 0xfe9, 0xff0, 0xff5, 0xff9, 0xffd, 0xffe,

0xfff, 0xffe, 0xffd, 0xff9, 0xff5, 0xff0, 0xfe9, 0xfe1,

0xfd8, 0xfcd, 0xfc2, 0xfb5, 0xfa7, 0xf98, 0xf87, 0xf76,

0xf63, 0xf4f, 0xf3a, 0xf24, 0xf0d, 0xef5, 0xedc, 0xec1,

0xea6, 0xe8a, 0xe6c, 0xe4e, 0xe2e, 0xe0e, 0xded, 0xdca,

0xda7, 0xd83, 0xd5f, 0xd39, 0xd12, 0xceb, 0xcc3, 0xc9a,

0xc71, 0xc47, 0xc1c, 0xbf1, 0xbc5, 0xb98, 0xb6b, 0xb3d,

0xb0f, 0xae0, 0xab1, 0xa82, 0xa52, 0xa22, 0x9f1, 0x9c0,

0x98f, 0x95e, 0x92c, 0x8fa, 0x8c8, 0x896, 0x864, 0x832,

0x800, 0x7cd, 0x79b, 0x769, 0x737, 0x705, 0x6d3, 0x6a1,

0x670, 0x63f, 0x60e, 0x5dd, 0x5ad, 0x57d, 0x54e, 0x51f,

0x4f0, 0x4c2, 0x494, 0x467, 0x43a, 0x40e, 0x3e3, 0x3b8,

0x38e, 0x365, 0x33c, 0x314, 0x2ed, 0x2c6, 0x2a0, 0x27c,

0x258, 0x235, 0x212, 0x1f1, 0x1d1, 0x1b1, 0x193, 0x175,

0x159, 0x13e, 0x123, 0x10a, 0xf2, 0xdb, 0xc5, 0xb0,

0x9c, 0x89, 0x78, 0x67, 0x58, 0x4a, 0x3d, 0x32,

0x27, 0x1e, 0x16, 0xf, 0xa, 0x6, 0x2, 0x1,

0x0, 0x1, 0x2, 0x6, 0xa, 0xf, 0x16, 0x1e,

0x27, 0x32, 0x3d, 0x4a, 0x58, 0x67, 0x78, 0x89,

0x9c, 0xb0, 0xc5, 0xdb, 0xf2, 0x10a, 0x123, 0x13e,

0x159, 0x175, 0x193, 0x1b1, 0x1d1, 0x1f1, 0x212, 0x235,

0x258, 0x27c, 0x2a0, 0x2c6, 0x2ed, 0x314, 0x33c, 0x365,

0x38e, 0x3b8, 0x3e3, 0x40e, 0x43a, 0x467, 0x494, 0x4c2,

0x4f0, 0x51f, 0x54e, 0x57d, 0x5ad, 0x5dd, 0x60e, 0x63f,

0x670, 0x6a1, 0x6d3, 0x705, 0x737, 0x769, 0x79b, 0x7cd};

int main(void)

{

clock_init(); // Sets Clock to 32MHz

daca_init(); // Digital to Analog Converter

tcc0_init(); // Frequency Timer

dmac_init(); // Initialize Table in Memory

// Enable audio amplifier (disable shutdown)

PORTC.DIRSET = PIN7_bm;

PORTC.OUTSET = PIN7_bm;

while (1)

{

// NO OP

}

}

void daca_init(void)

{

// Enable DAC Channel 1 for Speaker

DACA.CTRLB = DAC_CHSEL_SINGLE1_gc;

// Use AREFB (2.5V) as the DAC reference

DACA.CTRLC = DAC_REFSEL_AREFB_gc;

DACA.CTRLA = DAC_ENABLE_bm | DAC_CH1EN_bm | DAC_CH0EN_bm;

}

void tcc0_init(void)

{

TCC0.CNT = 0;

TCC0.INTFLAGS = TC0_OVFIF_bm;

// 32 MHz / ((83 + 1) * 256) = 1488.1 Hz

// Closest integer timer period to 1479.98 Hz

TCC0.PER = 83;

TCC0.CTRLA = TC_CLKSEL_DIV1_gc;

}

void dmac_init(void)

{

DMA.CTRL|= DMA_RESET_bm; // Reset DMA

DMA.CH0.TRFCNT = 512;

DMA.CH0.REPCNT = 0;

// Reload full block, increment source/output, reload in bursts

DMA.CH0.ADDRCTRL =

DMA_CH_SRCRELOAD_BLOCK_gc |

DMA_CH_SRCDIR_INC_gc |

DMA_CH_DESTRELOAD_BURST_gc |

DMA_CH_DESTDIR_INC_gc;

// Turn on Repeat, CH0, and 2BYTE burst

DMA.CH0.CTRLA =

DMA_CH_REPEAT_bm |

DMA_CH_SINGLE_bm |

DMA_CH_BURSTLEN_2BYTE_gc;

// Timer overflow -> Event Channel 0 -> DMA Update

EVSYS.CH0MUX = EVSYS_CHMUX_TCC0_OVF_gc;

DMA.CH0.TRIGSRC = DMA_CH_TRIGSRC_EVSYS_CH0_gc;

// Source Addressing thru pointers

DMA.CH0.SRCADDR0= (uint8_t)((uintptr_t) sine_table);

DMA.CH0.SRCADDR1= (uint8_t)(((uintptr_t) sine_table) >> 8);

DMA.CH0.SRCADDR2= (uint8_t)(((uint32_t)((uintptr_t)sine_table)) >>16);

// Destination Addressing

DMA.CH0.DESTADDR0= (uint8_t)((uintptr_t) &DACA.CH1DATA);

DMA.CH0.DESTADDR1= (uint8_t)(((uintptr_t) &DACA.CH1DATA) >> 8);

DMA.CH0.DESTADDR2= (uint8_t)(((uint32_t)((uintptr_t)&DACA.CH1DATA)) >>16);

DMA.CH0.CTRLA |= DMA_CH_ENABLE_bm;

DMA.CTRL = DMA_ENABLE_bm;

}

Lab8_5.c

/*

* lab8_5.c

*

* Created: 7/29/2026 8:53:41 AM

*  Author: Evan Baesler

*/

#include <avr/io.h>

// PROTOTYPES

extern void clock_init(void);

void usartd0_init(void);

void daca_init(void);

void tcc0_init(void);

void dmac_init(void);

char usartd0_in(void);

void set_note(float freq);

void set_wave(uint8_t mode);

uint16_t per; // Played Note Frequency

uint8_t mode = 0; // 0 = sine, 1 = triangle

// Baud Rate (9600bps from previous labs)

uint8_t BSEL = 207;

uint8_t BSCALE = 0;

// 256 Value Sine Wave Approximation

uint16_t sine_table[256] =

{0x800, 0x832, 0x864, 0x896, 0x8c8, 0x8fa, 0x92c, 0x95e,

0x98f, 0x9c0, 0x9f1, 0xa22, 0xa52, 0xa82, 0xab1, 0xae0,

0xb0f, 0xb3d, 0xb6b, 0xb98, 0xbc5, 0xbf1, 0xc1c, 0xc47,

0xc71, 0xc9a, 0xcc3, 0xceb, 0xd12, 0xd39, 0xd5f, 0xd83,

0xda7, 0xdca, 0xded, 0xe0e, 0xe2e, 0xe4e, 0xe6c, 0xe8a,

0xea6, 0xec1, 0xedc, 0xef5, 0xf0d, 0xf24, 0xf3a, 0xf4f,

0xf63, 0xf76, 0xf87, 0xf98, 0xfa7, 0xfb5, 0xfc2, 0xfcd,

0xfd8, 0xfe1, 0xfe9, 0xff0, 0xff5, 0xff9, 0xffd, 0xffe,

0xfff, 0xffe, 0xffd, 0xff9, 0xff5, 0xff0, 0xfe9, 0xfe1,

0xfd8, 0xfcd, 0xfc2, 0xfb5, 0xfa7, 0xf98, 0xf87, 0xf76,

0xf63, 0xf4f, 0xf3a, 0xf24, 0xf0d, 0xef5, 0xedc, 0xec1,

0xea6, 0xe8a, 0xe6c, 0xe4e, 0xe2e, 0xe0e, 0xded, 0xdca,

0xda7, 0xd83, 0xd5f, 0xd39, 0xd12, 0xceb, 0xcc3, 0xc9a,

0xc71, 0xc47, 0xc1c, 0xbf1, 0xbc5, 0xb98, 0xb6b, 0xb3d,

0xb0f, 0xae0, 0xab1, 0xa82, 0xa52, 0xa22, 0x9f1, 0x9c0,

0x98f, 0x95e, 0x92c, 0x8fa, 0x8c8, 0x896, 0x864, 0x832,

0x800, 0x7cd, 0x79b, 0x769, 0x737, 0x705, 0x6d3, 0x6a1,

0x670, 0x63f, 0x60e, 0x5dd, 0x5ad, 0x57d, 0x54e, 0x51f,

0x4f0, 0x4c2, 0x494, 0x467, 0x43a, 0x40e, 0x3e3, 0x3b8,

0x38e, 0x365, 0x33c, 0x314, 0x2ed, 0x2c6, 0x2a0, 0x27c,

0x258, 0x235, 0x212, 0x1f1, 0x1d1, 0x1b1, 0x193, 0x175,

0x159, 0x13e, 0x123, 0x10a, 0xf2, 0xdb, 0xc5, 0xb0,

0x9c, 0x89, 0x78, 0x67, 0x58, 0x4a, 0x3d, 0x32,

0x27, 0x1e, 0x16, 0xf, 0xa, 0x6, 0x2, 0x1,

0x0, 0x1, 0x2, 0x6, 0xa, 0xf, 0x16, 0x1e,

0x27, 0x32, 0x3d, 0x4a, 0x58, 0x67, 0x78, 0x89,

0x9c, 0xb0, 0xc5, 0xdb, 0xf2, 0x10a, 0x123, 0x13e,

0x159, 0x175, 0x193, 0x1b1, 0x1d1, 0x1f1, 0x212, 0x235,

0x258, 0x27c, 0x2a0, 0x2c6, 0x2ed, 0x314, 0x33c, 0x365,

0x38e, 0x3b8, 0x3e3, 0x40e, 0x43a, 0x467, 0x494, 0x4c2,

0x4f0, 0x51f, 0x54e, 0x57d, 0x5ad, 0x5dd, 0x60e, 0x63f,

0x670, 0x6a1, 0x6d3, 0x705, 0x737, 0x769, 0x79b, 0x7cd};

// 256 Value Triangle Wave Approximation

uint16_t tri_table[256] =

{0x00, 0x20, 0x40, 0x60, 0x80, 0xa0, 0xc0, 0xe0, 0x100,

0x120, 0x140, 0x160, 0x180, 0x1a0, 0x1c0, 0x1e0, 0x200,

0x220, 0x240, 0x260, 0x280, 0x2a0, 0x2c0, 0x2e0, 0x300,

0x320, 0x340, 0x360, 0x380, 0x3a0, 0x3c0, 0x3e0, 0x400,

0x420, 0x440, 0x460, 0x480, 0x4a0, 0x4c0, 0x4e0, 0x500,

0x520, 0x540, 0x560, 0x580, 0x5a0, 0x5c0, 0x5e0, 0x600,

0x620, 0x640, 0x660, 0x680, 0x6a0, 0x6c0, 0x6e0, 0x700,

0x720, 0x740, 0x760, 0x780, 0x7a0, 0x7c0, 0x7e0, 0x800,

0x81f, 0x83f, 0x85f, 0x87f, 0x89f, 0x8bf, 0x8df, 0x8ff,

0x91f, 0x93f, 0x95f, 0x97f, 0x99f, 0x9bf, 0x9df, 0x9ff,

0xa1f, 0xa3f, 0xa5f, 0xa7f, 0xa9f, 0xabf, 0xadf, 0xaff,

0xb1f, 0xb3f, 0xb5f, 0xb7f, 0xb9f, 0xbbf, 0xbdf, 0xbff,

0xc1f, 0xc3f, 0xc5f, 0xc7f, 0xc9f, 0xcbf, 0xcdf, 0xcff,

0xd1f, 0xd3f, 0xd5f, 0xd7f, 0xd9f, 0xdbf, 0xddf, 0xdff,

0xe1f, 0xe3f, 0xe5f, 0xe7f, 0xe9f, 0xebf, 0xedf, 0xeff,

0xf1f, 0xf3f, 0xf5f, 0xf7f, 0xf9f, 0xfbf, 0xfdf, 0xfff,

0xfdf, 0xfbf, 0xf9f, 0xf7f, 0xf5f, 0xf3f, 0xf1f, 0xeff,

0xedf, 0xebf, 0xe9f, 0xe7f, 0xe5f, 0xe3f, 0xe1f, 0xdff,

0xddf, 0xdbf, 0xd9f, 0xd7f, 0xd5f, 0xd3f, 0xd1f, 0xcff,

0xcdf, 0xcbf, 0xc9f, 0xc7f, 0xc5f, 0xc3f, 0xc1f, 0xbff,

0xbdf, 0xbbf, 0xb9f, 0xb7f, 0xb5f, 0xb3f, 0xb1f, 0xaff,

0xadf, 0xabf, 0xa9f, 0xa7f, 0xa5f, 0xa3f, 0xa1f, 0x9ff,

0x9df, 0x9bf, 0x99f, 0x97f, 0x95f, 0x93f, 0x91f, 0x8ff,

0x8df, 0x8bf, 0x89f, 0x87f, 0x85f, 0x83f, 0x81f, 0x800,

0x7e0, 0x7c0, 0x7a0, 0x780, 0x760, 0x740, 0x720, 0x700,

0x6e0, 0x6c0, 0x6a0, 0x680, 0x660, 0x640, 0x620, 0x600,

0x5e0, 0x5c0, 0x5a0, 0x580, 0x560, 0x540, 0x520, 0x500,

0x4e0, 0x4c0, 0x4a0, 0x480, 0x460, 0x440, 0x420, 0x400,

0x3e0, 0x3c0, 0x3a0, 0x380, 0x360, 0x340, 0x320, 0x300,

0x2e0, 0x2c0, 0x2a0, 0x280, 0x260, 0x240, 0x220, 0x200,

0x1e0, 0x1c0, 0x1a0, 0x180, 0x160, 0x140, 0x120, 0x100,

0xe0, 0xc0, 0xa0, 0x80, 0x60, 0x40, 0x20};

// Point to a Table

uint16_t *waveform_ptr = sine_table;

int main(void)

{

clock_init(); // Sets Clock to 32MHz

usartd0_init(); // Initialize USART

daca_init(); // Digital To Analog Converter

tcc0_init(); // Audio Playback Timer

tcc1_init(); // Audio Delay Timer

dmac_init(); // Initialize table in memory

// Enable audio amplifier (disable shutdown)

PORTC.DIRSET = PIN7_bm;

PORTC.OUTSET = PIN7_bm;

while (1)

{

if(USARTD0.STATUS & USART_RXCIF_bm) // Wait for USART to be ready

{

char c = usartd0_in(); // Get input character

if(c=='s' || c=='S') // Do both 's' or 'S' for caps insensitivity

{

mode ^= 1; // Flip between 0/1

set_wave(mode); // Update mode

}

else if(c=='e' || c=='E')

set_note(1046.50); // Play C6

else if(c=='4')

set_note(1108.73); // Play C#6 / Db6

else if(c=='r' || c=='R')

set_note(1174.66); // Play D6

else if(c=='5')

set_note(1244.51); // Play D#6 / Eb6

else if(c=='t' || c=='T')

set_note(1318.51); // Play E6

else if(c=='y' || c=='Y')

set_note(1396.91); // Play F6

else if(c=='7')

set_note(1479.98); // Play F#6 / Gb6

else if(c=='u' || c=='U')

set_note(1567.98); // Play G6

else if(c=='8')

set_note(1661.22); // Play G#6 / Ab6

else if(c=='i' || c=='I')

set_note(1760.00); // Play A6

else if(c=='9')

set_note(1864.66); // Play A#6 / Bb6

else if(c=='o' || c=='O')

set_note(1975.53); // Play B6

else if(c=='p' || c=='P')

set_note(2093.00); // Play C7

}

if(TCC1.INTFLAGS & TC1_OVFIF_bm) // Wait for delay to end

{

TCC1.INTFLAGS = TC1_OVFIF_bm; // Reset flag

TCC0.CTRLA = TC_CLKSEL_OFF_gc; // Turn off audio playback timer

}

}

}

void daca_init(void)

{

// Enable DAC Channel 1 for Speaker

DACA.CTRLB = DAC_CHSEL_SINGLE1_gc; // Select DAC Channel 1 (PA3) (Speaker)

// Use AREFB (2.5V) as the DAC reference

DACA.CTRLC = DAC_REFSEL_AREFB_gc;

DACA.CTRLA = DAC_ENABLE_bm | DAC_CH1EN_bm; // Enable CH1 DAC

}

void tcc0_init(void)

{

// Audio Playback Timer

TCC0.CNT = 0; // Reset TCC0 count

TCC0.INTFLAGS = TC0_OVFIF_bm; // Reset TCC0 flags

TCC0.CTRLA = TC_CLKSEL_OFF_gc; // Turn off playback

}

void tcc1_init(void)

{

// Audio Delay Timer

TCC1.CNT = 0;

// 275ms delay (just enough to prevent skipping when holding key)

TCC1.PER = (275e-3)*(32e6/256); // 256 PRE set in DMA

TCC1.INTFLAGS = TC1_OVFIF_bm; // Reset flags

TCC1.CTRLA = TC_CLKSEL_OFF_gc; // Turn off delay

}

void dmac_init(void)

{

DMA.CTRL|= DMA_RESET_bm; // Reset DMA

DMA.CH0.TRFCNT = 512; // 256 Data Entries * 2

DMA.CH0.REPCNT = 0; // Infinite Repeats

// Reload full block, increment source/output, reload in bursts

DMA.CH0.ADDRCTRL =

DMA_CH_SRCRELOAD_BLOCK_gc |

DMA_CH_SRCDIR_INC_gc |

DMA_CH_DESTDIR_INC_gc |

DMA_CH_DESTRELOAD_BURST_gc;

// Turn on repeat, CH0, and 2BYTE burst

DMA.CH0.CTRLA =

DMA_CH_REPEAT_bm |

DMA_CH_SINGLE_bm |

DMA_CH_BURSTLEN_2BYTE_gc;

// Timer overflow -> Event Channel 0 -> DMA Update

EVSYS.CH0MUX = EVSYS_CHMUX_TCC0_OVF_gc;

DMA.CH0.TRIGSRC = DMA_CH_TRIGSRC_EVSYS_CH0_gc;

// Source addressing thru pointers

DMA.CH0.SRCADDR0= (uint8_t)((uintptr_t) sine_table);

DMA.CH0.SRCADDR1= (uint8_t)(((uintptr_t) sine_table) >> 8);

DMA.CH0.SRCADDR2= (uint8_t)(((uint32_t)((uintptr_t)sine_table)) >>16);

// Destination Addressing

DMA.CH0.DESTADDR0= (uint8_t)((uintptr_t) &DACA.CH1DATA);

DMA.CH0.DESTADDR1= (uint8_t)(((uintptr_t) &DACA.CH1DATA) >> 8);

DMA.CH0.DESTADDR2= (uint8_t)(((uint32_t)((uintptr_t)&DACA.CH1DATA)) >>16);

DMA.CH0.CTRLA |= DMA_CH_ENABLE_bm;

DMA.CTRL = DMA_ENABLE_bm;

}

void usartd0_init(void)

{

// TX (PD3) output

PORTD.OUTSET = PIN3_bm;

PORTD.DIRSET = PIN3_bm;

// RX (PD2) input

PORTD.DIRCLR = PIN2_bm;

// 9600 baud @ 32 MHz

USARTD0.BAUDCTRLA = BSEL;

USARTD0.BAUDCTRLB = BSCALE;

// 8 data bits, no parity, 1 stop bit

USARTD0.CTRLC = USART_CHSIZE_8BIT_gc;

// Enable transmitter / receiver

USARTD0.CTRLB = USART_TXEN_bm | USART_RXEN_bm;

}

char usartd0_in(void)

{

while (!(USARTD0.STATUS & USART_RXCIF_bm)); // Wait for USART

return USARTD0.DATA; // Sends back char from USARTD0 input

}

void set_note(float freq)

{

per = (32000000.0 / (256.0 * freq)) - 1; // PER = SCF/(PRE*FREQ) - 1

TCC0.PER = per; // Update frequency

TCC0.CNT = 0; // Reset count

TCC0.CTRLA = TC_CLKSEL_DIV1_gc; // Start audio playback timer

TCC1.CNT = 0;

TCC1.INTFLAGS = TC1_OVFIF_bm;

TCC1.CTRLA = TC_CLKSEL_DIV256_gc; // Start 275ms delay timer

}

void set_wave(uint8_t mode)

{

// Temporarily disable DMA channel during config

DMA.CH0.CTRLA &= ~DMA_CH_ENABLE_bm;

if(mode == 0) // Mode 0 = Sine

{

waveform_ptr = sine_table;

}

else // Mode 1 = Tri

{

waveform_ptr = tri_table;

}

// Update source address

DMA.CH0.SRCADDR0 = (uint8_t)((uintptr_t)waveform_ptr);

DMA.CH0.SRCADDR1 = (uint8_t)(((uintptr_t)waveform_ptr) >> 8);

DMA.CH0.SRCADDR2 = (uint8_t)(((uint32_t)((uintptr_t)waveform_ptr)) >> 16);

// Reset transfer count

DMA.CH0.TRFCNT = 512;

// Re-enable DMA CH0

DMA.CH0.CTRLA |= DMA_CH_ENABLE_bm;

}

void note_timer(void)

{

TCC1.CNT = 0; // Reset timer

TCC1.INTFLAGS = TC1_OVFIF_bm; // Reset flags

TCC1.CTRLA = TC_CLKSEL_DIV256_gc; // Enable 275ms timer

}

APPENDIX

Figure 1: 1.6V Output from DAC in Part 1

Figure 2: E5 Waveform

Figure 3: Attempt at 1059.26 Hz (WITH ISR), fails at 1kHz

Figure 4: Max Frequency Sine Wave in lab8_2.2a (NO ISR) of 2.23kHz.

Figure 5: Lab8_2b Waveform of F# (6) or 1479.98 Hz WITHOUT DMA

Figure 6: Lab6_3 Waveform of F# (6) or 1479.98 Hz WITH DMA

Figure 7: Program Memory Addressing

Figure 8: Data Memory Addressing

Figure 9: Ports Register Summary

Figure 10: Timer/Counter Register Summary

Figure 11: Interrupts Register Summary

Figure 12: EBI Register Summary

Figure 13: EBI Chip Select Register Summary

Figure 14: PORTH Alternate Functions

Figure 15: PORTJ Alternate Functions

Figure 16: PORTK Alternate Functions

Figure 17: USART Register Summaries

Figure 18: PORTC Alternate Functions

Figure 19: PORTF Alternate Functions

Figure 20: SPI Register Summary

Figure 21: Axis Directions for IMU