Q1:

%% Second Order Impulse Response Plot

clear; clc; close all;

% HW Parameters

w_0 = 1;

alphas = [0, 0.2, 1, 5];

numerator = 1;

%% Calculate Four Impulse Responses

% Empty Plot

figure();

hold on;

% Our function is 1/(s^2 + 2 * alpha + w_0^2)

% So, our denominator coefficients are: 1, 2 * alpha, w_0^2

for i = 1:length(alphas)

alpha = alphas(i);

denominator = [1 2*alpha w_0^2];

H = tf(numerator, denominator); % tf creates a transfer function from our numerator and denominator.

impulse(H); % takes the impulse of our tf

end

xlabel('time (s)');

ylabel('Amplitude');

title('Impulse Response for Second-Order Systems');

axis([0, 50, -1, 1]);

legend('\alpha = 0', '\alpha = 0.2', '\alpha = 1', '\alpha = 5');

grid on;

hold off;

%% Calculate Step Responses

% Empty Plot 2

figure(2);

hold on;

for i = 1:length(alphas)

alpha = alphas(i);

denominator = [1 2*alpha w_0^2];

H = tf(numerator, denominator);

step(H); % takes the step of our tf

end

xlabel('time (s)');

ylabel('Amplitude');

title('2nd Order Step Response');

axis([0, 50, -1, 2])

legend('\alpha = 0', '\alpha = 0.2', '\alpha = 1', '\alpha = 5');

grid on;

hold off;

Q2:

%% Delayed Unit Step Response

clear; clc; close all;

% HW Parameters

w_0 = 1;

alpha = 0.2*w_0;

dt = 0.1;

a = 5;

t = 0:dt:50;

w_d = sqrt(w_0^2 - alpha^2);

% Unit Step

u = double(t >= a);

% Impulse Response

h = (1/w_d)*exp(-alpha*t).*sin(w_d*t);

% Exact Response

x_exact = zeros(size(t));

tau = t-a;

idx = t >= a;

x_exact(idx) = (1/w_0^2)*(1 - exp(-alpha*tau(idx)) .* (cos(w_d*tau(idx)) + (alpha/w_d)*sin(w_d*tau(idx))));

% Numerical Response

x = conv(u,h)*dt;

x_numerical = x(1:length(t));

% Plot

figure;

plot(t,u,t,x_exact,t,x_numerical);

xlabel('Time (s)');

ylabel('Amplitude');

title('Delayed Unit Step Response');

legend('u(t-a)','x exact','x numerical');

axis([0 50 0 2]);

grid on;