REQUIREMENTS NOT MET

N/A

VIDEO FILE LINK

https://youtu.be/lf14VnVAcK0?si=UlVwC0SdvP7_AuV1

PROBLEMS ENCOUNTERED

I had Soor help me through Slack and realized that I was doing POS wrong, I was assuming that we were saying a variable was true when it was 1, but through his help found that is only true for SOP, for POS we look for where the variables are 0 to be true.

FUTURE WORK/APPLICATIONS

This is the most practical so far, 7-segment LEDs are used all around the world, for appliances, watches, anything that can count, etc. You could use these 7-segment decoder to convert from binary to hexadecimal, drive a display, which are fundamental skills in electronics.

PRE-LAB QUESTIONS OR EXERCISES

SECTION 1

Q1:

Table 1: Truth Table for 2-input MUX

| S0 | D0 | D1 | Y |
| --- | --- | --- | --- |
| 0 | 0 | - | 0 |
| 0 | 1 | - | 1 |
| 1 | - | 0 | 0 |
| 1 | - | 1 | 1 |

Row 1: (/S0 * D0) = 1

Row 3: (S0 * D1) = 1

SOP: Y = (/S0 * D0) + (S0 * D1)

Row 2: (/S0 + /D0) = 0

Row 4: (S0 + /D1) = 0

POS: Y = (/S0 + /D0) * (S0 + /D1)

Table 2: K-Map for 2-input MUX.

| (S0)/(D0*D1) | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | 1 |
| 1 | 0 | 1 | 1 | 0 |

Groupings

For Red: /S0 + /D0  Zeroes

For Orange: /S0 * D0  Ones

For Green: S0 + /D1  Zeroes

For Blue: S0 * D1  Ones

SOP: Y = (/S0*D0) + (S0*D1)  Ones

POS: Y = (/S0 + /D0) * (S0 + /D1)  Zeroes

ALREADY SIMPLIFIED!

Table 3: Truth Table for 2-input MUX.

| D0 | D1 | S0 |  | /S0 | (/S0*D0) | S0*D1 | Y |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 |  | 1 | 0 | 0 | 0 |
| 0 | 0 | 1 |  | 0 | 0 | 0 | 0 |
| 0 | 1 | 0 |  | 1 | 0 | 0 | 0 |
| 0 | 1 | 1 |  | 0 | 0 | 1 | 1 |
| 1 | 0 | 0 |  | 1 | 1 | 0 | 1 |
| 1 | 0 | 1 |  | 0 | 0 | 0 | 0 |
| 1 | 1 | 0 |  | 1 | 1 | 0 | 1 |
| 1 | 1 | 1 |  | 0 | 0 | 1 | 1 |

Table 4: Voltage Table for 2-input MUX.

| D0(L) | D1(H) | S0(H) |  | Y(L) |
| --- | --- | --- | --- | --- |
| L | L | L |  | L |
| L | L | H |  | H |
| L | H | L |  | L |
| L | H | H |  | L |
| H | L | L |  | H |
| H | L | H |  | H |
| H | H | L |  | H |
| H | H | H |  | L |

Figure 1: MUX Functional Block Diagram & Circuit Block Diagram

Figure 2: Quartus Wiring Diagram for Y = (/S0 * D0) + (S0 * D1)

Figure 3: Annotated Waveform for Y = (/S0 * D0) + (S0 * D1)

Figure 4: Switch & LED Diagram (at HHH.)

SECTION 2

Table 5: Truth Table for 4-input MUX with wildcards.

| S0 | S1 | D0 | D1 | D2 | D3 | Y |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | - | - | - | 0 |
| 0 | 0 | 1 | - | - | - | 1 |
| 0 | 1 | - | 0 | - | - | 0 |
| 0 | 1 | - | 1 | - | - | 1 |
| 1 | 0 | - | - | 0 | - | 0 |
| 1 | 0 | - | - | 1 | - | 1 |
| 1 | 1 | - | - | - | 0 | 0 |
| 1 | 1 | - | - | - | 1 | 1 |

Row 2: /S0 * /S1 * D0 = 1

Row 4: /S0 * S1 * D1 = 1

Row 6: S0 * /S1 * D2 = 1

Row 8: S0 * S1 * D3 = 1

SOP  Y = (/S0*/S1*D0) + (/S0 * S1 * D1) + (S0 * /S1 * D2) + (S0 * S1 * D3)

Row 1: (/S0 + /S1 + /D0) = 0

Row 3: (/S0 + S1 + /D1) = 0

Row 5: (S0 + /S1 + /D2) = 0

Row 7: (S0 + S1 + /D3) = 0

POS  Y = (/S0 + /S1 + /D0) * (/S0 + S1 + /D1) * (S0 + /S1 + /D2) * (S0 + S1 + /D3)

Table 6: Voltage Table for 4-input MUX with wildcards.

| S0(H) | S1(H) | D0(L) | D1(H) | D2(H) | D3(H) | Y(H) |
| --- | --- | --- | --- | --- | --- | --- |
| L | L | L | - | - | - | H |
| L | L | H | - | - | - | L |
| L | H | - | L | - | - | L |
| L | H | - | H | - | - | H |
| H | L | - | - | L | - | L |
| H | L | - | - | H | - | H |
| H | H | - | - | - | L | L |
| H | H | - | - | - | H | H |

Figure 5: Functional Block Diagram and Circuit Design for 4-input MUX.

Figure 6: Quartus Circuit Diagram for a 4-input MUX.

Figure 7: Quartus Waveform Simulation for 4-input MUX.

Figure 8: LED & Switch Legend for 4-input MUX.

SECTION 3

Table 7: Truth Table for Hex to 7-Segment Decoder

| T | X3 | X2 | X1 | X0 | A | B | C | D | E | F | G | LEGEND |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 |
| 0 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 |
| 0 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 2 |
| 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 3 |
| 0 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 4 |
| 0 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 5 |
| 0 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 6 |
| 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 7 |
| 0 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 8 |
| 0 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 9 |
| 0 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | A |
| 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | b |
| 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | C |
| 0 | 1 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | d |
| 0 | 1 | 1 | 1 | 0 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | E |
| 0 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | F |
| 1 | - | - | - | - | 1 | 1 | 1 | 1 | 1 | 1 | 1 | TEST |

SOP(A):

T + (/T*/X0*/X1*/X2*/X3) + (/T*/X0*X1*/X2*/X3) + (/T*X0*X1*/X2*/X3) + (/T*X0*/X1*X2*/X3) + (/T*/X0*X1*X2*/X3) + (/T*X0*X1*X2*/X3) + (/T*/X0*/X1*/X2*X3) + (/T*X0*/X1*/X2*X3) + (/T*/X0*X1*/X2*X3) + (/T*/X0*/X1*X2*X3) + (/T*/X0*X1*X2*X3) + (/T*X0*X1*X2*X3)

POS(A):

(T+/X0+X1+X2+X3)*(T+X0+X1+/X2+X3)*(T+/X0+/X1+X2+/X3)*(T+/X0+X1+/X2+/X3)

= T*(/X0+X1+X2+X3)*(X0+X1+/X2+X3)*(/X0+/X1+X2+/X3)*(/X0+X1+/X2+/X3)

SOP(B):

T + (/T*/X0*/X1*/X2*/X3) + (/T*X0*/X1*/X2*/X3) + (/T*/X0*X1*/X2*/X3) + (/T*X0*X1*/X2*/X3) + (/T*/X0*/X1*X2*/X3) + (/T*X0*X1*X2*/X3) + (/T*/X0*/X1*/X2*X3) + (/T*X0*/X1*/X2*X3) + (/T*/X0*X1*/X2*X3) + (/T*X0*/X1*X2*X3)

POS(B):

(T + /X0 + X1 + /X2 + X3) * (T + X0 + /X1 + /X2 + X3) * (T + /X0 + /X1 + X2 + /X3) * (T + X0 + X1 + /X2 + /X3) * (T + X0 + /X1 + /X2 + /X3) * (T + /X0 + /X1 + /X2 + /X3)

= T * (/X0 + X1 + /X2 + X3) * (X0 + /X1 + /X2 + X3) * (/X0 + /X1 + X2 + /X3) * (X0 + X1 + /X2 + /X3) * (X0 + /X1 + /X2 + /X3) * (/X0 + /X1 + /X2 + /X3)

SOP(C):

T + (/X0*/X1*/X2*/X3) + (X0*/X1*/X2*/X3) + (X0*X1*/X2*/X3) + (/X0*/X1*X2*/X3) + (X0*/X1*X2*/X3) + (/X0*X1*X2*/X3) + (X0*X1*X2*/X3) + (/X0*/X1*/X2*X3) + (X0*/X1*/X2*X3) + (/X0*X1*/X2*X3) + (X0*X1*/X2*X3) + (X0*/X1*X2*X3)

Table 8: Voltage Table for 4-input MUX

| T(L) | X3(H) | X2(H) | X1(H) | X0(H) | A(L) | B(L) | LEGEND |
| --- | --- | --- | --- | --- | --- | --- | --- |
| L | - | - | - | - | L | L | TEST |
| H | L | L | L | L | L | L | 0 |
| H | L | L | L | H | H | L | 1 |
| H | L | L | H | L | L | L | 2 |
| H | L | L | H | H | L | L | 3 |
| H | L | H | L | L | H | L | 4 |
| H | L | H | L | H | L | H | 5 |
| H | L | H | H | L | L | H | 6 |
| H | L | H | H | H | L | L | 7 |
| H | H | L | L | L | L | L | 8 |
| H | H | L | L | H | L | L | 9 |
| H | H | L | H | L | L | L | A |
| H | H | L | H | H | H | H | b |
| H | H | H | L | L | L | H | C |
| H | H | H | L | H | H | L | d |
| H | H | H | H | L | L | H | E |
| H | H | H | H | H | L | H | F |

Figure 9: Annotated 7-Segment Display Quartus Waveform Simulation.

Figure 10: Switch & LED Legends for Hex to 7-Segment Display

SECTION 4

Figure 11: Quartus Diagram of 74153 Testing

Figure 12: Test of 74'153 in Quartus Waveform.

Formula: F = W(/X + XYZ) + /(/W + Z)

Table 9: Truth Table for F = W(/X+XYZ)+/(/W+Z)

| W | X | Y | Z | /X | XYZ | /X + XYZ | W(/X+XYZ) | /W | /W+Z | /(/W+Z) | F |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 0 | 0 |
| 0 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 1 | 0 | 0 |
| 0 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 1 | 0 | 0 |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 |
| 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 |
| 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 0 |
| 1 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
| 1 | 0 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 1 |
| 1 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
| 1 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |
| 1 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |
| 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 1 |

SOP:

(W*/X*/Y*/Z)+(W*/X*/Y*Z)+(W*/X*Y*/Z)+(W*/X*Y*Z)+(W*X*/Y*/Z)+(W*X*Y*/Z)+(W*X*Y*Z)

POS:

(W+X+Y+Z)*(W+X+Y+/Z)*(W+X+/Y+Z)*(W+X+/Y+/Z)*(W+/X+Y+Z)*(W+/X+Y+/Z)*(W+/X+/Y+Z)*(W+/X+/Y+/Z)*(/W+/X+Y+/Z)

SOP K-MAP

Table 10: K-Map to simplify function F

| WX \ YZ | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 | 0 | 0 | 0 | 0 |
| 01 | 0 | 0 | 0 | 0 |
| 11 | 1 | 0 | 1 | 1 |
| 10 | 1 | 1 | 1 | 1 |

W*/Z

W*/X

W*Y

F = W(/X+Y+/Z)

Table 11: Truth Table for Simplified F = W(/X+Y+/Z)

| W | X | Y | Z | /X | /Z | /X+Y+/Z | F |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 | 1 | 1 | 0 |
| 0 | 0 | 0 | 1 | 1 | 0 | 1 | 0 |
| 0 | 0 | 1 | 0 | 1 | 1 | 1 | 0 |
| 0 | 0 | 1 | 1 | 1 | 0 | 1 | 0 |
| 0 | 1 | 0 | 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 1 | 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 1 | 1 | 0 | 1 | 1 |
| 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 |
| 1 | 0 | 1 | 1 | 1 | 0 | 1 | 1 |
| 1 | 1 | 0 | 0 | 0 | 1 | 1 | 1 |
| 1 | 1 | 0 | 1 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 |
| 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 |

Table 12: Voltage Table for Simplified F = W(/X+Y+/Z)

| W(L) | X(H) | Y(H) | Z(H) | F(H) |
| --- | --- | --- | --- | --- |
| L | L | L | L | H |
| L | L | L | H | H |
| L | L | H | L | H |
| L | L | H | H | H |
| L | H | L | L | H |
| L | H | L | H | L |
| L | H | H | L | H |
| L | H | H | H | H |
| H | L | L | L | L |
| H | L | L | H | L |
| H | L | H | L | L |
| H | L | H | H | L |
| H | H | L | L | L |
| H | H | L | H | L |
| H | H | H | L | L |
| H | H | H | H | L |

Figure 13: Quartus Diagram for F = W(/X+Y+/Z)

Figure 14: Annotated Quartus Waveform for 4-input MUX and F = W(/X+Y+/Z)