Full Name: Evan Baesler

EEL 3135	– Classwork #04

Milestone:

• Complete any 1 question and 2 parts of a second question to receive credit.

Question #1:	Consider

.

- Convert each cosine’s angular frequency to Hz and list f1,f2. Determine if x(t) is periodic by checking if f1 = K1f0 and f2 = K2f0 for integers K1,K2 with gcd(K1,K2) = 1. Report f0, ω0, T0.

240π→120Hz, 360π→180Hz

GCD(120,180) = 60 Hz

f0=60Hz, ω0=120π, T0=160 seconds.

120 = 60*2, where 2 is our k value

180 = 60*3, where 3 is our k value

GCD(2,3) = 1

- Append a third term 4cos(600πt). Re-test periodicity and recompute f0, ω0, T0 if periodic. Comment on how adding terms can break or restore periodicity.

GCD(120,180,300) = 60 Hz

f0=120Hz, ω0=240π, T0=1120 seconds.

120 = 60 * 2

180 = 60 * 3

300 = 60 * 5

GCD(2,3,5) = 1

- MATLAB: Write a script that:

  - Defines x(t) over t ∈ [0,4T0] with fs = 20kHz,

  - Plots x(t) and visually verifies periodicity over multiple periods (add grid, labels),

  - Overlays vertical lines every T0.

- Briefly explain why choosing a smaller valid f0 (when possible) can change a “non-periodic” result to a periodic one.

Question #2:	Sum/Product & Amplitude Modulation

Let fc = 400 Hz and ∆ = 20 Hz:

.

- Use complex exponentials to show:

xsum(t) = 2cos(2π∆t)cos(2πfct).

Then state f0 conditions based on gcd(K − 1,K + 1) where fc = K∆.

- MATLAB: Plot xsum(t) for t ∈ [0,0.5] s with fs = 10kHz. In separate figures, plot cos(2π∆t) and cos(2πfct). Label clearly and add legends.

- Let v(t) = 3 + 2.5cos(60πt) and x(t) = v(t)cos(2π · 150t). Expand x(t) into cosines (carrier + sidebands) and sketch/label the spectrum.

- MATLAB: Generate x(t) numerically and stem-plot its spectrum. Verify amplitudes match your algebra.

Question #3:	Orthogonality & Reconstruction

Consider the periodic signal

.

- Rewrite x(t) using inverse Euler to express , identify ω0, and list the nonzero ak.

- State the Fourier series coefficient definition:

and explain why orthogonality justifies this extraction.

- MATLAB: Compute ak numerically for |k| ≤ 6 using trapz over one period. Display a table of k vs. ak (real & imag parts). Compare with your analytic values.

- MATLAB: Make a two-sided stem plot of |ak| for |k| ≤ 6. Clearly label the nonzero coefficients.

- Comment briefly on why coefficients at unused harmonics are ≈ 0 (numerical orthogonality).

Question #4:	Fourier Coefficients, Spectrum & Reconstruction

Consider the periodic signal

.

- Express x(t) as a Fourier Series:

.

Compute all nonzero ak values by rewriting each term using inverse Euler’s formula and combining terms.

Fundamental Frequencies:0,π5,π3,π2

- GCD: π30

- π5=π30*6

- π3=π30*10

- π2=π30*15

- K= -15, -10, -6, 0, 6, 10, 15.

- 6cosπ5t-π6 Inverse Euler's:

  - =3ejπ5te-jπ6+3e-jπ5tejπ6→a±6

- -4cosπ3t+π4 Inverse Euler's:

  - =-2ejπ3tejπ4-2e-jπ3te-jπ4→a±10

- 3sinπ2tInverse Euler's:

  - =-j1.5ejπ2t-j1.5e-jπ2t →a±15

- MATLAB: Create a two-sided stem plot of |ak| vs. k for |k| ≤ 6. Clearly mark k values where |ak| = 0.

- Using the ak coefficients, reconstruct an approximation of x(t):

.

x1=5+6cosπ5t-π6-4cosπ3t+π4+3sinπ2t

x2=5+6cosπ5t-π6

Fundamental frequency = π30

Plot both x(t) and ˆx(t) on the same graph over t ∈ [0,2T0] and comment on how well they match.

- Construct sinusoids from the following Fourier coefficients ak.

a0 = 2,

a−1 = 3,

a1 = 3,

a−4 = −4j,

a4 = 4j	(fundamental frequency: ω0 = 40)

x(t) = 2+6cos40t-8sin⁡(160t)